Speed-of-Sound Measurements and a Fundamental Equation of State for Propylene Glycol

Tim Eisenbach1, Christian Scholz1, Roland Span1, Diego Cristancho2, Eric W. Lemmon3, Monika Thol1

1Ruhr-Universität Bochum, Thermodynamics, 44801 Bochum, Germany

2Dow, Lake Jackson, 77566 Texas, USA

3National Institute of Standards and Technology, Applied Chemicals and Materials Division, Boulder, 80305 Colorado, USA

Abstract

A fundamental equation of state was developed for propylene glycol. It is written in terms of the Helmholtz energy with the independent variables temperature and density. Due to its fundamental nature, it can be used to calculate all thermodynamic state properties from the Helmholtz energy and its derivatives with respect to the independent variables. Special attention was paid not only to accurately reproduce the available experimental data, but also to correct extrapolation. Therefore, this equation can be used for the application in mixture models. For the development of the present equation of state, the available literature data were supplemented with new experimental speed-of-sound measurements, which were conducted in the temperature range from (293.2 to 353.2) K with pressures up to 20 MPa. High accuracy was achieved by applying the well-established double-path length pulse-echo technique and a careful sample preparation.

Key words:

equation of state, experimental measurements, extrapolation, Helmholtz energy, propylene glycol, speed of sound

_____________

*corresponding author, e-mail: t.eisenbach@thermo.ruhr-uni-bochum.de

Content

List of Tables	3

List of Figures	4

1	Introduction	6

2	Characteristics of Propylene Glycol	7

3	Speed-of-Sound Measurements	9

3.1	Apparatus description	9

3.2	Calibration	10

3.3	Measurement fluid	11

3.4	Experimental procedures	12

3.5	Results	13

4	Equation of State	16

4.1	Ideal contribution	16

4.2	Residual part	19

5	Fitting Procedure	20

6	Critical Point Determination	22

7	Isobaric Heat Capacity of the Ideal Gas	26

8	Equation of State for Propylene Glycol	27

9	Comparison with Literature Data and Validation of the Equation of State	30

9.1	Comparisons of homogeneous density data	31

9.2	Comparisons of vapor-liquid equilibrium data	36

9.3	Comparisons of caloric properties	40

10	Extrapolation and Physical Behavior	45

11	Conclusion	51

12	References	53

# List of Tables

Molar mass and characteristic thermodynamic properties of propylene glycol, and the universal gas constant.	8

Sample information of the used experimental materials.	12

Speed-of-sound measurements in propylene glycol carried out with the pulse-echo sensor.	13

Uncertainty budget for the speed-of-sound measurements in propylene glycol.	14

Overview of all experimentally obtained and estimated critical properties of propylene glycol.	24

Parameters of the Planck-Einstein terms of the ideal part of the equation of state according to Eq. (17).	27

Parameters of the residual part of the equation of state for propylene glycol according to Eq. (18).	28

Test values for the verification of computer implementation.	28

Parameters of the ancillary equations for the vapor pressure pv, the saturated liquid density ρ′, and the saturated vapor density ρ′′ according to Eqs. (19) to (21).	29

Summary of all underlying experimental and estimated data points.	30

Summary of available homogeneous liquid density data along with the temperature and pressure ranges and their overall average absolute relative deviations (AAD) with respect to the equation of state.	31

Summary of vapor-pressure (pv) and saturated liquid density (ρ′) data along with the temperature ranges and their average absolute relative deviations (AAD) with respect to the equation of state.	36

Summary of all liquid-phase speed-of-sound data along with the temperature and pressure ranges and their average absolute relative deviations (AAD) with respect to the equation of state.	40

Summary of all isobaric heat-capacity data along with the temperature range and their overall average absolute relative deviations (AAD) with respect to the equation of state.	43

# List of Figures

- Results of the calibration speed-of-sound measurements in pure water. Relative deviations of the experimental speeds of sound wDATA from values calculated with the equation of state of Wagner and Pruß17 wEOS at 0.1 MPa are plotted versus temperature.	11

Speed-of-sound measurements in propylene glycol as function of pressure along four isotherms.	13

Deviations of the calculated ideal-gas isobaric heat-capacity data from the equation of state.	26

Deviations of values calculated with the ancillary equations for the vapor pressure pv, the saturated liquid density ρ', and the saturated vapor density ρ'' from the equation of state.	29

p,T-diagram with all available data in relation to the vapor-pressure curve and the critical point.	31

Deviations of homogeneous liquid density data from the equation of state for selected data sets in selected temperature ranges as a function of the logarithmic pressure.	33

Deviations of homogeneous liquid density data of Bridgman52 from the equation of state as a function of the logarithmic pressure.	34

Deviations of homogeneous liquid density data from the equation of state at atmospheric pressure as a function of temperature.	35

Deviations of all available vapor-pressure data from the equation of state as a function of temperature.	39

Deviations of the available speed-of-sound data in the liquid phase from the equation of state in selected temperature ranges as a function of the logarithmic pressure along with the speed-of-sound data measured in this work.	41

Deviations of the available speed-of-sound data in the liquid phase from the equation of state at atmospheric pressure as a function of temperature.	42

Deviations of all available liquid-phase isobaric heat-capacity data from the equation of state as a function of temperature.	44

Ideal curves for the equation of state for propylene glycol (PGC) in terms of the reduced temperature T/Tc and pressure p/pc: Vapor-pressure curve (pv), Boyle curve (BL), Joule-Thomson inversion curve (JT), Joule inversion curve (JI), and the ideal curve (ID).	46

p,ρ-diagram along isotherms up to 106 K (top). T,ρ-diagram showing the phase boundaries, the rectilinear diameter, and the critical isobar (bottom).	47

Second (B), third (C), and fourth (D) viral coefficients.	47

Speed of sound along isobars up to 120 MPa (in steps of 8 MPa) as a function of temperature.	48

Residual isochoric heat capacity along isobars up to 50 MPa (in steps of 5 MPa) as a function of temperature.	48

Phase identification parameter as a function of temperature along selected isobars.	49

Residual Grüneisen parameter as a function of temperature along selected isobars.	50

# Introduction

In consideration of today’s climate issues, optimization of facilities in process engineering and power generation becomes more and more important, in particular the highly accurate investigation and understanding of the utilized fluids in process applications. Even though there is a vast quantity of fluids with a comprehensive data basis, in some fluid states (e.g. at high pressures and temperatures) experimental data with reasonable uncertainties are not easily to obtain with conventional measurements. To maintain the access to highly precise fluid properties over a broad range of the fluid states, equations of state can be developed to provide reference thermodynamic properties, which is also of major importance for fluids with a less comprehensive data basis.

This work provides an equation of state for propylene glycol, which represents the available literature data with an accuracy that is in line with future requirements in mixture modelling. Propylene glycol is often used to lower the freezing point of water, e.g., when being applied as de-icing fluids for aircrafts. Furthermore, it is frequently employed as a substitute for ethylene glycol in mixtures used in refrigerant processes because it is less toxic.1 A somewhat more specific application of propylene glycol is its use as an inhibitor of gas hydrate formation. The effect of hydrate formation poses a high safety but also economic risk when drilling holes, e.g., for the use of offshore and deep-water reservoirs in the context of carbon capture and storage.2

In the course of the development of thermodynamic models for CO2-rich mixtures, the fluid-phase models3 were subsequently supplemented by phase equilibria calculation methods including solids4–6 and hydrates.7–9 This mixture model will further be extended with propylene glycol as a suitable candidate for retarding hydrate formation. For this purpose, a fundamental equation of state is needed to represent the thermodynamic properties of pure propylene glycol at fluid states, which was developed in this work.

A reliable basis of thermal and caloric properties over a broad range of fluid states is essential in fitting equations of state with high demand on validity. To fulfill this need, additional speed-of-sound data were measured with very low experimental uncertainties. In the course of the development of a fundamental equation of state, it is also of special importance to maintain physically correct behavior with a reasonable number of terms.

# Characteristics of Propylene Glycol

Propylene glycol (C3H8O2, CAS No. 57-55-6), also known as 1,2-propanediol, is a synthetically produced organic compound. With its two hydroxyl groups, it can be assigned to the group of diols. The pure compound is a viscous and highly hygroscopic liquid and is miscible with water, acetone, and chloroform. Propylene glycol is not toxic for humans or animals due to its harmless metabolic byproducts, mainly consisting of lactic or pyruvic acid.10 The center carbon atom of the propylene glycol structure is seen as chiral or asymmetric because of its four different bonds, and determines the number of two enantiomers of propylene glycol. Propylene glycol is most commonly manufactured from the noncatalytic hydrolysis of propylene oxide under pressures of 2 MPa and temperatures from (120 to 190) °C. The broad application as an antifreeze is based on the cryoprotective properties of the pure compound as well as the aqueous solution. These properties are related to the absence of any crystal-like structure in the glassy state up to concentrations of 68 wt. % propylene glycol in an aqueous solution.11 The glass transition behavior and, by implication, the glass transition temperature TG are dependent on cooling or warming rates, how the glassy state was formed, and the amount of contained water. An aqueous solution of 45 wt. % liquid propylene glycol changes to glassy at -107.5 °C with a cooling rate of 2.5 K per minute.11 According to its antifreeze properties, propylene glycol is often applied in cooling processes as a low-temperature heat transfer fluid. Furthermore, accompanied by its non-toxic character, propylene glycol is broadly applied in the food industry, pharmaceuticals, and cosmetics.

Propylene glycol molecules have strong associative interactions due to their two hydroxyl groups. Since propylene glycol is highly hygroscopic, it is often found in an aqueous solution where it shows a clear affinity to be hydrated, rather than to be self-associated. New approaches in assessing association of propylene glycol by Rhys et al.12 yield a peak of hydroxyl group interactions between two molecules in a 30 mol. % aqueous solution at intermolecular or group distances of around 1.95 Å. Taking this into account, pure propylene glycol, without any contribution of hydration, is likely to have a strong associative character. Due to its hygroscopic character, propylene glycol and glycols in general are used in the natural gas industry in water removing processes.

Although propylene glycol is an important fluid in a wide range of applications, it is still not sufficiently investigated. In order to develop mixture models, often involving water or carbon dioxide, highly accurate calculations of properties from a reliable equation of state for pure propylene glycol are needed.

Modern fundamental equations of state are formulated in terms of the Helmholtz energy, which allows the calculation of all thermal and caloric properties directly from combinations of the partial derivatives of the Helmholtz energy with respect to the independent variables temperature and density. For the development of a fundamental equation of state, a sufficiently wide range of fluid states needs to be covered by experimental measurements. In this work, a new equation of state was developed under consideration of available literature data as well as our own measurements. Table 1 shows the thermodynamic and physical properties of propylene glycol, which are important for the development of its equation.

Table 1. Molar mass and characteristic thermodynamic properties of propylene glycol, and the universal gas constant.

| Physical Properties | Denotation | Value | Unit | Reference |
| --- | --- | --- | --- | --- |
| Critical temperature | Tc | 674.0 | K | This work |
| Critical density | ρc | 4.46 | mol dm–3 | This work |
| Critical pressure | pc | 7.2918 | MPa | This work |
| Normal-boiling-point temperature | TB | 461.224 | K | This work |
| Triple-point temperature | Ttr | 242.8 | K | Howard13 |
| Liquid density at triple point | ρtr,liq | 14.113 | mol dm–3 | This work |
| Glass-transition temperature | TG | 169.15a | K | Boutron and Kaufmann11 |
| Molar mass | M | 76.09442 | g mol–1 | Wieser and Berglund14 |
| Universal gas constant | R | 8.314462618 | J mol–1 K–1 | CODATA15 |
| a glass-transition temperature for pure propylene glycol at a warming rate of 2.5 K min–1 |  |  |  |  |

# Speed of Sound Measurements

The new equation of state for propylene glycol presented here was not only fitted to available literature data, but also to speed-of-sound data measured in this work. The well established double-path length pulse-echo technique was utilized to conduct those measurements in the temperature range from (293.2 to 353.2) K at pressures up to 20 MPa. This data set is supposed to confirm the speed-of-sound measurements in pure propylene glycol that were carried out by Dávila et al.16 in similar ranges of temperature and pressure.

## Apparatus description

The speed-of-sound measurements were carried out with the double-path-length pulse-echo technique. The apparatus was set up by Gedanitz et al.17 according to the design of Meier and Kabelac18 and was optimized by Wegge19 and Wegge et al.20 These publications provide more detailed information on the measuring technique and the apparatus. As a brief explanation, the acoustic sensor consists of two polished stainless steel reflectors with an eccentrically attached piezoelectric x-cut quartz crystal. Two different path lengths of L1 = 20 mm and L2 = 30 mm appear. The piezoelectric quartz transducer is excited at its carrier frequency of 8 MHz, with a waveform generator (Agilent Technologies, type: 33220A, USA), which compiles a 30-cycle sinusoidal burst that is further modulated by a half-cycle sine²-function. The ultrasonic pulse is coupled into the fluid and propagates until it reaches the two reflectors, and is returned to the quartz crystal that is simultaneously used as emitter and receiver. Due to the different path lengths, two echoes separated by a time difference Δtecho are captured by a digital oscilloscope (Agilent Technologies, type: MS6032A, USA). The waveform was averaged from 16 consecutive pulses and applied with a bandpass filter based on a Fast-Fourier transformation with a bandwidth of 1.6 MHz in order to enhance the signal-to-noise ratio. The time difference Δtecho between the first and second echo is computed on the basis of the processed data at each state point according to the algorithm described by Dubberke et al.,21 while the two path lengths are assumed to be constant for a series of measurements. The speed of sound can be determined by the following equation:

| w=2∙L2-L1∆techo+τ. | (1) |
| --- | --- |

According to Harris,22 a correction value τ is considered in order to evaluate the time difference between the ideal-plane wave and the real case. The determination of this correction is described in detail by Gedanitz.23

The acoustic sensor comprising the quartz transducer and the two reflectors is housed in a stainless steel pressure vessel. A glass feed-through is implemented to connect the sensor within the pressure vessel and the electronic devices outside. The entire pressure vessel is immersed into a calibration bath thermostat (Fluke, type: 7060) filled with an ethylene glycol + water mixture. Stable temperature conditions in the range from (293 to 353) K can therefore be guaranteed. The temperature inside the pressure vessel is measured by a long-stem 25 Ω standard platinum resistance thermometer (SPRT, Rosemount Aerospace, type: 162CE, USA) calibrated on the ITS-90, and connected to a direct current thermometry bridge (Isotech, type: TTI-2, UK). The standard uncertainty for the temperature measurement was u(T) = 0.004 K. The pressure measurement is implemented with two vibrating-quartz-crystal pressure transducers (Paroscientific, type: 1000-500A and 1000-6K, USA) that cover different ranges of pressure. The utilization of a differential pressure indicator (Rosemount, type: 3051, USA) to separate the measuring cell and the pressure measuring circuit is crucial to avoid contamination of the pressure transducers with the respective liquid sample. The standard uncertainty for the pressure measurement was u(p) = 0.0024 MPa. Both the pressure transducers and the differential pressure indicator are tempered by a second circulation bath thermostat, which operates with deionised water.

## Calibration

In order to determine the speed of sound according to Eq. (1), the time difference between two echoes is measured at each state point. In addition, the difference between the two path lengths needs to be accurately known for a series of measurements, since it has a direct impact on the measurement result. However, changes in path length could occur with varying temperature and pressure due to thermal expansion or compression. For those reasons, calibration with purified water was performed to determine the actual difference in path lengths, which is discussed in detail in a previous work by Wegge et al.20 Water is a suitable fluid for the calibration procedure since its speed of sound is well known over a wide temperature and pressure range due to highly reliable literature data such as the data sets by Del Grosso and Mader24 and Fujii and Masui,25 along with an available reference equation of state with very low uncertainty from Wagner and Pruß.26 Speed-of-sound measurements in high-purity water were carried out at ten state points at T = (274.2, 278.2, 283.2, 293.2, 303.2, 313.2, 323.2, 333.2, 343.2, and 353.2) K and ambient pressure. Relative deviations of the measured speed of sound in water from values calculated with the IAPWS-95 formulation of Wagner and Pruß26 are plotted versus temperature in Fig. 1. With the adjusted path length difference at reference temperature and one adjusted thermal expansion coefficient linear in temperature, all experimental speed-of-sound data are within the uncertainty of this equation of state, which was estimated to be 0.005 % from the equation of state of Wagner and Pruß26 in the observed temperature range. In addition the speed-of-sound data of water measured in the scope of the present work are in good agreement with the reference data sets of Fujii and Masui25 and Del Grosso and Mader24 and show a similar trend compared to the IAPWS-95 formulation (see Fig. 1).

Fig. 1. Results of the calibration speed-of-sound measurements in pure water. Relative deviations of the experimental speeds of sound wDATA from values calculated with the equation of state of Wagner and Pruß26 wEOS at 0.1 MPa are plotted versus temperature. The equation of state is represented by the zero line and has a relative expanded uncertainty of ± 0.005 % (k = 2).

## Measurement fluid

The experimental samples are specified and described in Table 2. Water and propylene glycol are both supplied in glass bottles and need to be decanted into stainless steel sample cylinders, which can be attached to the sample manifold of the apparatus.

The sample of water was transferred to the sample cylinder at ambient conditions and was further degassed by several freeze-pump-thaw cycles. For this purpose, the sample cylinder is stepwise immersed into liquid nitrogen in order to freeze the liquid phase of water from the bottom to the top. At the same time, impurities that were involved into the decanting process such as nitrogen, argon, or oxygen remain gaseous and detach from the liquid into the upper part of the cylinder. Those gaseous substances are removed by applying a vacuum to the system. The freeze-pump-thaw cycles are repeated until a steady vacuum pressure is reached.

Since propylene glycol has a hygroscopic behavior, the transfer of the sample could not be carried out at ambient conditions. Therefore, the decanting of the propylene glycol sample was performed within the inert, dry atmosphere of a glove-box (Fa. MBraun, Type: UNIlab Plus), to avoid contact with atmospheric humidity. Subsequently, the sample of propylene glycol was degassed according to the procedure described above.

The measurement fluid was selected with impurities as low as possible, since the quality and accuracy of the obtained speed-of-sound data as well as the related uncertainty of the measurements is significantly dependent upon the purity of the sample fluid.

The purity of the propylene glycol sample was stated by the supplier (Fa. Sigma-Aldrich) to be at least as good as 99.96 peak area-%, investigated by gas chromatography analysis. However, a GC requires a calibration to particular standards of well-known compositions in order to provide a quantitative analysis, which had not been conducted here. Thus, the effect of typical impurities of the propylene glycol sample was investigated by assuming that the remaining fraction is entirely dominated by the respective impurity. Water was found to be the impurity that affects the results of the measurements the most and the purity of propylene glycol was determined to correspond to 99.84 mol. %. Since no mixture model for the system propylene glycol + water is available in the literature, the contribution of the sample impurity to the experimental uncertainty was calculated with the reference equation of state of water26 and the present equation of state for propylene glycol combined with a linear mixing rule.

Table 2. Sample information of the used experimental materials.

| Chemical Name | Source | CAS Number | Purity / mole fraction | Further Purification | Analysis Method |
| --- | --- | --- | --- | --- | --- |
| Water | Sigma-Aldrich | 7732-18-5 | 0.999997 | Degassed | - |
| Propylene Glycol | Sigma-Aldrich | 57-55-6 | 0.9984 | Degassed | Gas Chromatography |

## Experimental procedures

The propylene glycol sample is filled in a cylinder (see Sec. 3.3) that is attached to the sample manifold of the apparatus. In order to remove any remaining substances in the speed-of-sound apparatus, the whole system was evacuated through a cooling trap for several days, with the set-point temperature of the bath thermostat set to the highest possible temperature of 353 K. To achieve a reasonable filling of the apparatus, a heating cable was used to heat up the sample cylinder to approximately 308 K while the bath thermostat was set to the lowest possible temperature of 253 K. Thus, the filling of the evacuated system could be realized by taking advantage of the temperature and pressure gradient as well as by support of a hand pump. Subsequently, the temperature of the bath thermostat was set to the lowest measuring temperature at 293 K and the hand pump was used to increase the pressure up to the highest desired value of the first state point. It was not possible to conduct measurements below a temperature of 293 K due to the increasing sound absorption of propylene glycol with decreasing temperature, which led to a damping of the acoustic wave. Measurements were performed at equilibrium. The speed-of-sound measurements were carried out along isotherms, starting with the lowest temperature at 293 K and the highest pressure (20 MPa). The adjustment of the following state point was reached by decreasing the pressure with the aid of the hand pump over a two-hour equilibration time. After the completion of one isotherm, the temperature was increased, followed by increasing the pressure with the hand pump to the highest possible value. Measurements were conducted over a temperature range from (293.2 to 353.2) K at pressures up to 20 MPa.

## Results

The speed of sound in propylene glycol was measured over the temperature range from (293.2 to 353.2) K at pressures up to 20 MPa. The measurements were conducted along four isotherms at (293.2, 313.2, 333.2 and 353.2) K with six state points each at (20.0, 15.0, 10.0, 5.0, 2.5 and 0.5) MPa, which led to a total of 24 (p, w, T) data points. At each state point, three measurements were carried out over a period of one hour in order to confirm the measurements and to investigate the short-time repeatability.

Fig. 2. Speed-of-sound measurements in propylene glycol as function of pressure along four isotherms. Lines are only shown to guide the eye.

In Fig. 2, the speed of sound is plotted versus pressure. State points with the same temperature are connected to visualize the course of each isotherm. The speed of sound decreases with increasing temperature and decreasing pressure. Furthermore, the course of the four isotherms is nearly linear and parallel to each other and there are no intersections among them. The numerical values of the measured sound speeds are listed in Table 3, including temperature, pressure, and relative combined, extended uncertainty (k = 2). Further discussions of the results with comparison to the equation of state and to other data from the literature such as the data sets of Zorębski et al.27 and Dávila et al.16 are given in Sec. 9.3.

Table 3. Speed-of-sound measurements in propylene glycol carried out with the pulse-echo sensor.

| T / K | p / MPa | w / (m s−1) | Ur(w) / % |
| --- | --- | --- | --- |
| 293.202 | 20.020 | 1593.294 | 0.02978 |
| 293.202 | 15.053 | 1576.429 | 0.02978 |
| 293.202 | 10.069 | 1558.840 | 0.02979 |
| 293.202 | 5.070 | 1540.767 | 0.02979 |
| 293.202 | 2.545 | 1531.412 | 0.02979 |
| 293.203 | 0.535 | 1523.903 | 0.02979 |
| 313.202 | 19.982 | 1541.057 | 0.02979 |
| 313.202 | 15.057 | 1523.365 | 0.02979 |
| 313.202 | 10.020 | 1504.612 | 0.02979 |
| 313.202 | 5.153 | 1485.924 | 0.02980 |
| 313.202 | 2.573 | 1475.843 | 0.02980 |
| 313.202 | 0.560 | 1467.805 | 0.02980 |
| 333.204 | 19.984 | 1488.229 | 0.02979 |
| 333.204 | 15.030 | 1469.455 | 0.02980 |
| 333.204 | 10.080 | 1450.003 | 0.02980 |
| 333.204 | 5.082 | 1429.738 | 0.02981 |
| 333.204 | 2.588 | 1419.358 | 0.02981 |
| 333.204 | 0.542 | 1410.713 | 0.02982 |
| 353.204 | 20.008 | 1434.907 | 0.02980 |
| 353.204 | 15.054 | 1415.063 | 0.02981 |
| 353.204 | 10.062 | 1394.357 | 0.02981 |
| 353.204 | 5.060 | 1372.816 | 0.02982 |
| 353.204 | 2.575 | 1361.835 | 0.02983 |
| 353.204 | 0.588 | 1352.880 | 0.02983 |

The relative combined expanded uncertainty (k = 2) for the measurements in propylene glycol was estimated to be 0.030 % following the approach of previous works by Wegge19 and Wegge et al.20 The uncertainty budget, including temperature and pressure measurements, sound-speed measurements, and short-term repeatability as well as impurities of the material (see Sec. 3.3), is shown in Table 4. The influence of the impurities was the main contribution to the uncertainty and was mainly caused by water, which differs significantly in speed of sound. The supplier also reported water to be the main impurity, and the influence on the uncertainty of the sound speed measurements was conservatively estimated to be 0.024 %.

Table 4. Uncertainty budget for the speed-of-sound measurements in propylene glycol.

| Source of uncertainty | Expanded uncertainty | Distribution | Coverage factor | Sensitivity coefficient | Standard uncertainty |
| --- | --- | --- | --- | --- | --- |
| Sound-speed measurement | 0.010 % | Normal | 2 | 1 | 0.0050 % |
| Short-term repeatability | 0.002 % | Normal | 1 | 1 | 0.0020 % |
| Temperature measurement | 8 mK | Normal | 2 | (∂w/∂T)pa | 0.0001 % |
| Pressure measurement | 0.00492 MPa | Normal | 2 | (∂w/∂p)Ta | 0.0001 % |
| Impurities | 0.024 % | Rectangular |  | 1 | 0.0140 % |
| Relative combined expanded uncertainty (k = 2): |  |  |  |  | 0.0298 % |
| a Calculated with the equation of state developed in the scope of this work. The maximum value has been used since the partial derivatives are approximately constant over the investigated temperature and pressure ranges |  |  |  |  |  |

# Equation of State

The fundamental equation of state is formulated in terms of the reduced Helmholtz energy, which is a continuous function over the entire fluid region including phase-equilibrium states. The reduced Helmholtz energy α can be derived from its molar form a

|  | ατ, δ=aT, ρRT | (2) |
| --- | --- | --- |

with δ=ρρc and τ=TcT as its independent variables. The molar Helmholtz energy is reduced by temperature and the molar gas constant R and is further separated into an ideal (superscript “o”) and a residual (superscript “r”) part

|  | ατ, δ=αoτ, δ+αrτ, δ. | (3) |
| --- | --- | --- |

The ideal part αo describes the hypothetical ideal-gas behavior in the limit of zero density, while the residual part αr describes the real-fluid effects deviating from the ideal-gas behavior, taking the intermolecular interactions into account.

## Ideal Contribution

In the limit of very low density, the real gas behavior is accurately described by the ideal gas model, since intermolecular forces as well as the molecule’s volume can be neglected. This limiting behavior can be described with the independence of the internal energy U from the volume V

|  | ∂Uo∂VT=0. | (4) |
| --- | --- | --- |

Thermal properties can be calculated from the ideal-gas law given by

|  | pv=RT. | (5) |
| --- | --- | --- |

This limiting behavior can be observed for many real gases at moderate and elevated temperatures and low pressures and is, thus, valid for broad technical applications. Caloric quantities are calculated from the ideal-gas law in combination with its ideal-gas heat capacity. The dynamic state of a molecule, which can be seen as the internal energy on a macroscopic scale, is responsible for the ideal-gas heat capacity and can be determined from the molecule’s intramolecular energies (e.g., translational, rotational, vibrational). In order to consider the various contributions to the ideal-gas heat capacity independently, a few simplifications and assumptions are usually made when developing an equation of state. Detailed information on the so-called “rigid-rotator, harmonic oscillator” model is given by Lucas28 and summarized in the following. To a considerable degree of accuracy, coupling effects between rotation and vibration within the molecule are negligible. It is further assumed that the whole mass is concentrated in the atomic nuclei and that the masses of the electrons can, therefore, be neglected. This assumption permits a separate investigation of the electronic energy.

The contribution of translation depends on the atom’s translational degrees of freedom, which are determined by three spatial directions. For temperatures greater than 10‒10 K, all three degrees are assumed to be fully excited. The translational energy of a molecule is originally derived from Schrödinger’s equation considering the likelihood of residence of one atom for various quantum numbers and three directions of motion. Since intermolecular forces are not considered in the ideal-gas model and molecules are assumed to be indistinguishable, the subdivision of the system’s total translational energy is permissible and, accordingly, so is the summation of all partial energy states to the molecular partition function. The molecular partition function is connected to the thermodynamic function of the system’s internal energy, which yields, for the translational contribution to the isochoric heat capacity,

|  | cv,transoR=32. | (6) |
| --- | --- | --- |

The contribution of the rotational energy to the ideal-gas heat capacity is derived from the kinetic energy of rotation in terms of the principal moments of inertia. Simplifying the molecule’s rotational kinetics by considering it to be rigid yields good agreement with reality at normal temperatures. For temperatures much higher than the characteristic temperature of rotation in a linear molecule, the contribution of rotational energy is independent of molecular species and temperature. The characteristic temperatures of external rotation28 in a non-linear molecule are formulated with regard to the associated axes of rotation and are anti-proportional to the related moments of inertia. For a non-linear molecule like propylene glycol, moments of inertia are not vanishing and thus contribute to the molecular partition function of external rotation28. Each mode is assumed to be fully excited at temperatures far beyond the characteristic temperature of rotation, which is in the order of a few 10 K depending on the investigated substance. Therefore, it contributes 12 R to the ideal-gas heat capacity. The non-linear propylene glycol molecule is excitable in three rotational directions and, thus, yields

|  | cv,rotoR=32. | (7) |
| --- | --- | --- |

During vibration, the atoms vibrate around their equilibrium position at which the intramolecular energy reaches its minimum. The intramolecular energy only depends on the distance between atoms and can, thus, be associated with intramolecular work, determining the frequency of the vibrational motion28. To simplify the quantum mechanical analysis, the model of the harmonic oscillator is used to describe the vibration of the diatomic type. Lucas28 provides further information about the relation between the energy states of a linear harmonic oscillator and the vibrational quantum numbers of a diatomic molecule. In the consideration of polyatomic molecules, the complex vibrational structure is transferred to a number of diatomic vibrational motions, introducing the normal coordinate analysis. The intramolecular potential energy can be written as a sum of quadratic terms as a function of the normal coordinates. Ultimately, it becomes possible to describe the vibrational behavior with a maximum number of 3n‒6 independent vibrations of the diatomic type, where n is the number of atoms in the molecule. The 3n‒6 normal modes are related to the fundamental harmonic frequencies, which determine the characteristic temperature of the i-th normal mode

|  | θvib,i=hvo,ikB, | (8) |
| --- | --- | --- |

where vo,i is the fundamental harmonic frequency of the i-th normal mode and h and kB are Planck’s and Boltzmann’s constants, respectively. The molecular partition function, from which the vibrational contribution to the ideal-gas heat capacity can be obtained, is related to the i-th characteristic temperature and is defined as the temperature-dependent part of the ideal-gas heat capacity:

|  | qvib,i=vexp-vθvib,iT=11-exp-θvib,iT. | (9) |
| --- | --- | --- |

Solving the thermodynamic function for the vibrational contribution of the ideal-gas isochoric heat capacity in Eq. (9) yields:

|  | cv,viboR=i=1IPEmiθvib,iT2expθvib,iTexpθvib,iT-12. | (10) |
| --- | --- | --- |

Eq. (10) is known as the Planck-Einstein form, which includes the temperature dependence of the ideal-gas heat capacity. The parameter mi considers the contribution of vibrational frequencies with similar characteristic temperatures. The characteristic temperature of the vibrational motions of the molecule θvib,i as well as the parameter mi can be determined spectroscopically, but are still treated as empirical parameters in the development of equations of state in order to reduce complexity. Eq. (10) is often expanded empirically by simple temperature-dependent monomial terms. These terms are usually used for fitting very limited data of the isochoric heat capacity of the ideal gas, but are omitted below because of the absence of data.

Taking all three contributions into account, the isobaric heat capacity for the ideal gas yields

|  | cpoR=cvoR+1=c0+i=1IPEmiθvib,iT2expθvib,iTexpθvib,iT-12, | (11) |
| --- | --- | --- |

where c0=4.0 refers to translational and rotational contributions for a non-linear molecule. The isochoric heat capacity of the ideal gas is directly related to the second derivative of the reduced Helmholtz energy with respect to the reduced temperature τ:

|  | ∂2αo∂τ2δ=-cvoRτ2. | (12) |
| --- | --- | --- |

Twofold integration of Eq. (12) yields the Helmholtz energy of the ideal gas:

|  | aoτ, δ=cΙΙ+cΙτ+(c0-1)lnτ+ln⁡(δ)+i=IPol+1IPol+IPEmiln1-exp-θiTc. | (13) |
| --- | --- | --- |

The integration constants cΙ and cΙΙ describe the caloric properties at the reference state in the Helmholtz-energy equation and its first derivative with respect to τ according to Span.29 The origin of the density dependence ln⁡(δ) lies in the formulation of the reduced Helmholtz energy, which is calculated from the internal energy, and the density dependent entropy. The reference state can arbitrarily be defined. For propylene glycol, the normal boiling point is chosen. At the saturated liquid state and TB(p = 1 atm) the enthalpy and entropy are set to zero considering both the contribution of the ideal part (see Eq. (13)) and the residual part of the reduced Helmholtz energy (see Eq. (14)).

## Residual part

The residual part of the Helmholtz energy describes real fluid behavior differing from the ideal-gas part. Although certain thermodynamic properties are related to distinct terms of the residual part, its parameters are treated empirically. Generally, the residual part can be defined as

|  | αrτ, δ=i=1IPolniτtiδdi+i=IPol+1IPol+IExpniτtiδdiexp-δpi+i=IPol+IExp+1IPol+IExp+IGBSniτtiδdiexp-ηiδ-εi2-βiτ-γi2. | (14) |
| --- | --- | --- |

The density exponents di and pi have to be positive integers, since derivatives with respect to density have to vanish for ρ→0, which represents an ideal gas with no molecular interaction aside from fully elastic collisions. That means that the density exponents are directly related to the virial coefficients, which are defined in the limit of zero density. The temperature exponents ti should be positive to have an infinite contribution to the Helmholtz energy near 0 K.

Polynomial (Pol) and exponential (Exp) terms are sufficient to describe the fluid behavior for most states. Nonetheless, those terms fail near the critical region to give the required accuracy. Thus, additional terms are needed to both yield larger gradients of derivatives of the Helmholtz energy and leave the non-critical region unaffected. For this purpose, Gaussian bell-shaped (GBS) terms were first introduced by Haar et al.30 and later adjusted by Setzmann and Wagner31 with remarkable accuracy in representing thermal and caloric properties in the critical region. The number of Gaussian bell-shaped terms is usually kept in the range of three to seven.

# Fitting Procedure

Fitting a fundamental equation of state is a process that adjusts the equation parameters so that experimental data are reproduced as accurately as possible while maintaining a physically correct behavior of the equation of state. To reach this aim, the influence of each parameter on certain thermodynamic properties is of major interest for the correlator, as well as the consideration of rough guidelines for temperature and density exponents, as explained in Sec. 4.2.

Reducing flexibility of the functional form, such as limiting the number of terms, can help to obtain suitable physical behavior and reasonable extrapolation. On the other hand, reducing the number of adjustable parameters may hamper the representation of the experimental data. Therefore, a compromise has to be found to fulfill both requirements. To obtain a highly accurate equation of state, it is important to adjust different properties over a broad range of temperature and pressure. The quality of the data sets used is most important, since all properties (e.g., speed of sound and heat capacity) are directly linked and, thus, influence the behavior of other properties. The equation of state should ideally represent all data within their experimental uncertainties. Since measurements are difficult to carry out at low temperatures or very high temperatures and pressures, as well as in the critical region, constraints can be used to ensure correct physical extrapolation of the equation of state.

The difference is the way how experimental data are used and correlated. The advantage of nonlinear fitting is the ability to use all properties as an input, without any transformation, whereas with linear fitting, preliminary equations are needed to obtain the independent variables of density and temperature (in case of the Helmholtz energy) from measured properties such as pressure and temperature. Furthermore, any parameter such as the power in polynomials or the parameters in exponential functions can directly be adjusted. Finally, nonlinear fitting procedures can control the physical behavior of any calculated thermodynamic property, since constraints, including inequality criteria, can be used.

In the process of developing the equation of state for propylene glycol, an error function is used, which differs from the least-squares algorithm. The input objective function f is

|  | f =Wx∙Fx | (15) |
| --- | --- | --- |

and can be further mathematically modified. Fx incorporates the deviations between measurement and calculation of thermodynamic properties of each experimental state point (E), along with the contributions from constraints (C), limits (L) and boundaries of a fitted parameter (B). Wx is the weight of each data point (E, C, L, and B) used in the fitting process, and the deviation Fx is defined by

|  | Fx =xE,C,L,B-xEOSxE,C,L,B. | (16) |
| --- | --- | --- |

The common objective function in conventional minimization problems is the sum of squares. For the development of the present equation of state, it was modified by Gao and Lemmon32 so that not only the square power but also other mathematical functions of logarithmic or exponential forms can be used. The objective function shape can independently be defined for experimental data, constraints, limits, and boundaries, such that errors can be minimized and considered differently. In this way, there is higher flexibility when solving multidimensional minimization problems.

During the fitting procedure, the adjustable parameters of the equation of state are varied in order to reduce the overall sum of errors of the calculated properties. With a better knowledge of the uncertainties of each data set, applying higher weights forces the fitting procedure to more closely match particular data points that have low uncertainties. Although underlying data can be represented correctly by the equation of state, it might still fail close to phase boundaries or in the critical region due to incorrect physical behavior. Incorrect behavior can be easily detected by unreasonable shapes of the thermodynamic diagrams, for instance by abrupt changes in various isolines. Consequently, both the deviations of data points and paths of thermodynamic properties must be considered simultaneously. The use of constraints helps to control extrapolation by creating barriers for slopes, curvatures, or even higher derivatives, and by shaping the isotherms or saturation lines to their correct form. The challenge in applying constraints to obtain a specific shape lies with the assumption of the correct behavior of thermodynamic properties for the particular fluid being fitted in regions where no experimental data are available. To predict certain behavior of properties, the correlator requires a comprehensive knowledge on thermodynamic properties and possible fluid-specific characteristics. This requirement was one of the most challenging aspects in this work because the physical behavior of propylene glycol significantly differs from the behavior of other well-known fluids due to its special physical characteristics (e.g., strong association or chemical decomposition at temperatures below the critical point).

# Critical Point Determination

Significant ranges of propylene glycol even at temperatures below the critical point are not investigated experimentally because of the thermal decomposition of the fluid. According to Diaz et al.,33 decomposition due to oxidation reactions happens at temperatures between (400 and 600) K, resulting in acetone, acetaldehyde, formaldehyde, and carbon dioxide via carbon bond cleavage. Even at lower temperatures, some dissociation can be expected. Consequently, the critical properties are difficult to obtain with conventional measurement techniques.

A flow method for the determination of critical data for thermally unstable compounds like propylene glycol was developed by VonNiederhausern et al.34 Estimated critical parameters, obtained by Kazakov et al.35 (2010) and Carande et al.36 (2015) from a regression method based on quantitative structure-property relationships (QSPR), are available and considered in the subsequent discussion.

The measurement of critical temperatures of a thermally unstable compound with a static method requires an unreasonably fast heating rate of the sample to avoid thermal decomposition. The advantage of a flow measurement technique is that accurate temperature scans can be taken over a wide time frame, since the temperature bath can be heated smoothly at very low residence times of the samples. To obtain the critical temperature and pressure, several temperature scans close to the critical point must be evaluated. A single, subcritical temperature scan is characterized by a flat, horizontal path of the fluid temperature while simultaneously rising bath temperature, which indicates isothermal boiling at constant pressure. This behavior does not exist above the critical point, where the transition between liquid and gas is indiscernible. Evaluation of several temperature scans at different pressures narrows down the temperature and pressure range in which the critical point is located. The critical point is inferred by the temperature and pressure where isothermal boiling is no longer observed. By varying the flow rates, different critical temperatures can be observed with the estimation method described above since the degree of thermal decomposition depends on the residence time at high temperature. The reported critical temperature is the extrapolated value at zero residence time, where the extrapolation was obtained from a quadratic fit of the measured data. The numerical values for the critical temperature and pressure are associated with uncertainties of 0.3 K and 0.007 MPa according to VonNiederhausern et al.37 This seems to be small since VonNiederhausern et al.34 already assume an error in the interpretation of the temperature scans of 1 K and 2 % for the critical temperature and pressure. Other contributions such as calibration uncertainties or uncertainties from the mathematical extrapolation procedure also have to be considered. Therefore, the overall uncertainty for the critical data is expected to be significantly higher. Critical densities cannot be obtained with this flow method. The difficulty associated with the method of VonNiederhausern et al.34 is getting close to the critical point in the first place. Since this experiment yields measurements associated with rather high uncertainties, estimation methods are also considered here.

The most established group contribution (GC) method of Joback and Reid38 is applicable for a broad field of compounds. It is described in Poling et al.39 and based on an earlier GC method of Lydersen (1955). However, the applicability of such methods needs to be closely reviewed when determining properties for substances with strong associative character. Propylene glycol is such a substance, which forms clusters of several molecules due to hydrogen bonds. Common GC methods only consider intramolecular group effects. Thus, they might not be suitable for estimating properties of associating, cluster-forming substances. In fact, the critical temperature of propylene glycol, estimated by the method of Joback and Reid38 is 626.67 K (see Table 5), which is roughly 50 K below the measured temperature of VonNiederhausern et al.37

The principle of the quantitative structure-property relationship (QSPR) methods for the estimation of critical parameters is explained in detail by Kazakov et al.35 and Carande et al.36 They provide detailed information on the theoretical background, methodical procedure, and implementation on the basis of broad underlying data sets. Generally, all QSPR methods are based on a relation between a property of interest and the molecular numerical features derived theoretically from the individual chemical structure of monomers of the considered fluid.

The molecular numerical features are commonly called descriptors and their relationship to the chosen property is established through non-linear regression analysis from broad data collections (cf. Kazakov et al.35). The properties considered in the investigations of Kazakov et al.35 and Carande et al.36 are the critical temperature Tc, the ratio of the critical temperature and the critical pressure Tcpc, and the saturation reduced pressure pv,r at Tr=TTc = 0.7, which is commonly used for the calculation of the acentric factor ω. Kazakov et al.35 and Carande et al.36 used a support vector machines (SVM) methodology for their regression, since it offers several numerical and computational advantages over other methods. With the demand for a very precise adjustment of the SVR and kernel parameters, the data set is split into three parts: the training set, the validation set, and the testing set. First an initial model is generated with a reduced and selected data set (training set), which is then able to predict the data from the validation set most accurately. Once the optimal model parameters are obtained, the final model is generated with the combined training and validation set (testing set). Kazakov et al.35 also provide detailed information on potential model errors of QSPR methods. Accordingly, a significant uncertainty contribution can be caused by uncertainties of the underlying experimental data used to produce the model. In case the predicted properties lie beyond the available data, such as with the critical properties for propylene glycol, and also physical mechanisms such as cluster-forming association are dominating the properties of the fluid, the suitability of the model decreases. The reason for that is the increasing deviation from a monomer-like structure, which is the basis of the structure input. With increasing associative behavior, the physical mechanisms are predominantly characterized by mixtures of monomers, dimers, and larger clusters. In fact, Kazakov et al.35 named two example fluids, propylene carbonate and γ-butyrolactone, with outlier characteristics in the estimated critical temperatures, which are predominantly caused by association.

Considering propylene glycol as an associative and linear-structured fluid, the uncertainty caused by association results in a noticeable deviation of the QSPR-predicted values from the underlying experimental data obtained from VonNiederhausern et al.37 Taking into account that predictions for the critical temperature of propylene glycol need to be extrapolated about 200 K beyond the available data, the estimation uncertainty obtained from QSPR-SVM is expected to gradually increase towards the critical point. Compared to GC-based methods, QSPR-SVM models exhibit better performance in approaching the experimentally obtained critical parameters of propylene glycol (see Table 5). Nonetheless the predictions of Kazakov et al.35 and Carande et al.36, who used an enhanced number of descriptors and improved data sets, both lack in the accurate prediction of the critical parameters due to the unsuitable model basis for associating fluids.

All QSPR and GC based estimations of the critical parameters for propylene glycol are shown in Table 5 along with the experimentally obtained data of VonNiederhausern et al.37

The critical parameters of an equation of state define the shape of the saturated liquid and vapor curves as well as slopes and curvatures of, e.g., isotherms in a pressure-density plot. Therefore, the choice of the critical parameters has a significant influence on the representation of experimental densities. Since the critical temperature of VonNiederhausern et al.37 is the only measurement and it is somewhat confirmed by the QSPR value of Kazakov et al.,35 Tc = 676.4 K was used as starting point in the fitting procedure. During the fit, it was slightly modified so that the accurate density measurements of Sampson et al.40 could be reproduced more accurately and the critical density was simultaneously adjusted. The critical pressure was calculated from the present equation of state. The final values used for the present equation of state are listed in Tables 1 and 5.

Table 5. Overview of all experimentally obtained and estimated critical properties of propylene glycol. Indicated uncertainties are reported in the corresponding publications.

| Reference | method | Tc / K | pc / MPa | ρc / (mol dm–3) |
| --- | --- | --- | --- | --- |
| VonNiederhausern et al.37 (2000) | experimental | 676.4 | 5.941 | - |
| Kazakov et al.35 (2010) | QSPR | 673.2 ± 3.5 | 6.043 ± 0.209a | - |
| Carande et al.36 (2015) | QSPR | 666.8 ± 1.9 | 5.865 ± 0.336a | - |
| Joback and Reid38 (1987) | GC | 626.67b | 5.792b | 4.246b |
| Steele et al.41 (2002) | fitted | 676.4c | 6.75 | 4.048 |
| Dean42 (1999) | not stated | 625.15 | 6.08 | 4.219 |
| Yaws43 (1999) | not stated | 626.0 | 6.1 | 4.184 |
| Gallant44 (1968) | not stated | 624.15 | 6.095 | 4.218 |
| Wilding et al.45 (1991) | not stated | 625.6 | 6.07 | - |
| This work | fitted | 674.0 | 7.2918 | 4.460 |
| avalues for pc calculated with Tc and Tc/pc from Kazakov et al.35 and Carande et al.36 considering propagation of uncertainty bcalculation rules taken from VDI Wärmeatlas46 cvalue for Tc taken from VonNiederhausern et al.37 and used in the fitting procedure for pc and ρc |  |  |  |  |

# Isobaric Heat Capacity of the Ideal Gas

As mentioned in Sec. 4.1, the ideal part of the equation of state is required for the calculation of caloric properties. It is subject to the temperature-dependent part of the ideal-gas heat capacity, which is defined by the intramolecular forces of the molecule. The process of developing the ideal part of the equation of state for propylene glycol was particularly challenging, since no data for the ideal-gas isobaric heat capacity are available in the literature. Therefore, the group contribution (GC) method of Joback and Reid,38 presented in the VDI Wärmeatlas,46 was used to estimate values. However, due to the same limitations as explained in Sec. 6, the data turned out to be not useful for the fitting process.

In the course of new software implementations and model developments of the Thermo Data Engine (TDE)47 the authors for the TDE software development have made progress in predicting ideal-gas heat capacities at constant pressure for associating fluids. Although not yet published, the data set for the ideal-gas heat capacity of propylene glycol of Diky et al.48 is investigated with respect to its deviations from the present equation of state along with the data obtained from the group contribution method of Joback and Reid,38 see Fig. 3.

Fig. 3. Deviations of the calculated ideal-gas isobaric heat-capacity data from the equation of state.

The predictions of Diky et al.48 match the present equation of state only slightly better than the data calculated with the group contribution approach of Joback and Reid.38 Therefore, both data sets are considered as unsuitable for the development of the present equation of state. Another approach had to be used to derive the ideal gas part of the reduced Helmholtz energy.

# Equation of State for Propylene Glycol

The ideal part of the equation of state is based on the isobaric heat capacity of the ideal gas. The ideal-gas isobaric heat capacity is generally constructed with Planck-Einstein and monomial terms (cf. Eq. (13)). For modern equations of state, monomial terms are not considered anymore, due to challenging extrapolation behavior. The ideal part of the equation of state for propylene glycol is written as

|  | α∘τ, δ=cΙΙ+cΙτ+3lnτ+ln⁡(δ)+i=12miln1-exp-θiτTc. | (17) |
| --- | --- | --- |

The two integration constants are cI= 2.5839633256032 and cII= 1.45359225002898 were adjusted so that the molar enthalpy h0 = 0 and molar entropy s0 = 0 for the saturated liquid at the normal boiling point. The parameters of the Planck-Einstein terms in Eq. (17) are listed in Table 6.

Table 6. Parameters of the Planck-Einstein terms of the ideal part of the equation of state according to Eq. (17).

| i | mi | θi / K |
| --- | --- | --- |
| 1 | 5 | 1000 |
| 2 | 28 | 1330 |

The estimated ideal-gas isobaric heat-capacity data, obtained from the estimation methods, were not considered in the validation of the ideal part of the equation of state due to the issues discussed in Sec. 7. Therefore, the parameters were mainly fitted to reliable speed-of-sound data in the liquid state as well as isobaric heat-capacity data. This has proven to be a reliable method as described by Lemmon and Span49 and Thol et al.50 They provided the enthalpy of evaporation and its temperature dependence results from the Clausius-Clapeyron relation with sufficient accuracy.

The residual part of the equation of state for propylene glycol consists of six monomial, six exponential, and seven Gaussian bell-shaped (GBS) terms:

|  | αrτ, δ=i=16niτtiδdi+i=712niτtiδdiexp-δpi+i=1319niτtiδdiexp-ηiδ-εi2-βiτ-γi2. | (18) |
| --- | --- | --- |

All parameters of Eq. (18) are listed in Table 7.

Table 7. Parameters of the residual part of the equation of state for propylene glycol according to Eq. (18).

| i | ni |  | ti | di | pi | ηi | βi | γi | εi |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 4.6611538×10‒2 | 1.000 | 4 |  |  |  |  |  |
| 2 |  | 2.0273992 | 0.140 | 1 |  |  |  |  |  |
| 3 | − | 2.6048664 | 0.920 | 1 |  |  |  |  |  |
| 4 | − | 5.8592792×10‒1 | 1.254 | 2 |  |  |  |  |  |
| 5 |  | 2.9674050×10‒1 | 0.425 | 2 |  |  |  |  |  |
| 6 |  | 5.3863656×10‒2 | 0.688 | 3 |  |  |  |  |  |
| 7 | − | 7.8280924×10‒2 | 1.600 | 1 | 2 |  |  |  |  |
| 8 | − | 7.6968025×10‒1 | 2.230 | 3 | 2 |  |  |  |  |
| 9 |  | 1.3016359×10‒1 | 1.550 | 2 | 1 |  |  |  |  |
| 10 | − | 1.5287585×10‒2 | 0.900 | 7 | 1 |  |  |  |  |
| 11 | − | 1.0000015×10‒2 | 5.000 | 1 | 3 |  |  |  |  |
| 12 | − | 1.5002210×10‒1 | 3.000 | 2 | 2 |  |  |  |  |
| 13 | − | 2.4426526×10‒1 | 1.100 | 2 | 2 | 18.7 | 685.0 | 1.090 | 0.789 |
| 14 | − | 3.5673700×10‒2 | 1.000 | 2 | 2 | 18.7 | 1230.0 | 1.040 | 0.990 |
| 15 | − | 2.7150835×10‒1 | 1.500 | 1 | 2 | 1.860 | 2.280 | 1.050 | 0.981 |
| 16 |  | 1.2948298 | 2.440 | 1 | 2 | 0.630 | 0.130 | 1.500 | 1.004 |
| 17 | − | 1.7031454 | 2.370 | 1 | 2 | 0.830 | 0.070 | 1.430 | 0.698 |
| 18 |  | 1.7600461 | 1.770 | 1 | 2 | 1.278 | 1.090 | 1.130 | 0.808 |
| 19 | − | 1.0654478 | 2.280 | 1 | 2 | 0.450 | 0.130 | 2.110 | 0.810 |

Considering all underlying data, the lower temperature limit of the equation is the triple-point temperature Ttr = 242.8 K. The upper temperature Tmax = 680 K is chosen to be slightly above the critical temperature. The upper pressure limit of pmax = 350 MPa is determined by the data set of Guignon et al.51 Bridgman52 reported density measurements at much higher pressures up to 1177 MPa. They are not considered in the fitting process; however, the equation of state shows a correct extrapolation behavior to fairly high pressures, which is discussed in more detail in Sec. 10.

Table 8 contains test values for the verification of computer implementation of the equation of state.

Table 8. Test values for the verification of computer implementation.

| TK | ρmol dm-3 | pMPa | wm s-1 | cpJ mol-1 K-1 | hJ mol-1 | sJ mol-1 K-1 | aJ mol-1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400.0 | 0.001 | 0.0033220476 | 214.57321 | 158.29676 | 43040.574 | 119.95947 | −8265.2622 |
| 400.0 | 13 | 61.287909 | 1467.8267 | 227.48403 | −11727.204 | −38.665809 | −975.33462 |
| 500.0 | 0.01 | 0.041324887 | 237.58157 | 197.10614 | 60808.024 | 138.55317 | −12601.051 |
| 500.0 | 13 | 196.80128 | 1633.6809 | 252.37935 | 19730.659 | 8.1675599 | 508.31841 |
| 680.0 | 13 | 430.54903 | 1814.6100 | 280.86419 | 80765.269 | 81.467593 | −7751.8504 |

Ancillary equations for the vapor pressure pv, the saturated liquid density ρ', and the saturated vapor density ρ'' are used to estimate starting values for the iterative calculation of properties along the saturation line. Thus, the fitting process of the equation of state as well as the calculation of phase-equilibrium data are expedited perceptibly. The parameters of Eqs. (19) to (21) are listed in Table 9.

|  | lnpvpc=TcTi=15ni1-TTcki | (19) |
| --- | --- | --- |
|  | ρ'ρc=1+i=15ni1-TTcki | (20) |
|  | lnρ''ρc=i=16ni1-TTcki | (21) |

Figure 4 shows that deviations between the ancillary equations and the fundamental equation of state are less than 0.07 %. Deviations are higher very close to the critical point.

Table 9. Parameters of the ancillary equations for the vapor pressure pv, the saturated liquid density ρ', and the saturated vapor density ρ'' according to Eqs. (19) to (21).

|  | pv, Eq. (19) |  | ρ', Eq. (20) |  | ρ'', Eq. (21) |  |
| --- | --- | --- | --- | --- | --- | --- |
| i | ni | ki | ni | ki | ni | ki |
| 1 | –0.1012×102 | 1.0 | 0.4600×100 | 0.21 | –0.20507×101 | 0.32 |
| 2 | 0.3150×101 | 1.5 | 0.2060×101 | 0.43 | –0.68362×101 | 0.90 |
| 3 | –0.5600×101 | 2.6 | 0.7430×100 | 2.70 | –0.19835×102 | 2.50 |
| 4 | –0.3370×100 | 4.0 | –0.1905×101 | 3.70 | –0.10097×102 | 4.20 |
| 5 | –0.2390×101 | 5.0 | 0.1536×101 | 4.70 | –0.55772×102 | 5.70 |
| 6 |  |  |  |  | –0.14455×103 | 12.00 |

Fig. 4. Deviations of values calculated with the ancillary equations for the vapor pressure pv, the saturated liquid density ρ', and the saturated vapor density ρ'' from the equation of state.ρ'ρ''

# Comparison with Literature Data and Validation of the Equation of State

The percent deviation of any measured or estimated data point xData from the calculated property xEOS, obtained from the present equation of state, is defined as

|  | ∆x=100xData-xEOSxData. | (22) |
| --- | --- | --- |

A convenient quantity to compare data sets from different sources with respect to their deviations from the equation of state is the averaged absolute relative deviation (AAD):

|  | AAD=1Ni=1N∆xi. | (23) |
| --- | --- | --- |

In Eq. (23), the quantity N corresponds to the number of data points for each author and property.

The available data for developing the equation of state for propylene glycol were found in 145 publications with measured data as well as estimations. Overall, 1366 data points are available, comprising homogeneous densities, saturated liquid densities, vapor pressures, isobaric heat capacities, sound speeds, and critical parameters. A summary of all data points and the quantity of fitted data points is given in Table 10.

Table 10. Summary of all underlying experimental and estimated data points.

| Property | All available data points | Fitted data points |
| --- | --- | --- |
| cp | 73 | 12 |
| cpo | 472a | - |
| pρT | 867 | 27 |
| ρ' | 17 | 1 |
| pv | 246 | 5 |
| w | 163 | 20 |
| Total without (cpo) | 1366 | 65 |
| a estimations of the ideal-gas heat capacity of Diky et al.48 and Joback and Reid38 |  |  |

In order to ensure stable and physically correct behavior of the equation of state, the underlying data sets need to represent the fluid states in a broad temperature and pressure range. The quality of data sets is closely related to the quality and the reliability of the equation of state. For the accuracy of the data, the measurement techniques, the purity of the sample, and the uncertainty of the employed apparatus is of great importance. Ideally, reliable uncertainties of the measured quantities are stated in the corresponding publications and can be included in the fitting procedure. Unfortunately, older publications usually lack such quantifications, but are not significantly less likely to contain high-quality data.

Figure 5 illustrates the limited data situation for propylene glycol. Homogeneous thermal and caloric data are restricted to the liquid phase and at temperatures far below the critical state due to the inaccessibility of the gaseous states in experiments at rather low pressures or the thermal decomposition of the sample at elevated temperatures.

Fig. 5. p,T-diagram with all available data in relation to the vapor-pressure curve and the critical point.pT

## Comparisons of homogeneous density data

The density data for propylene glycol are limited to homogeneous liquid densities within a relatively narrow temperature range, predominantly at ambient pressure, see Table 11. Figure 5 gives an overview of the location of the data in the fluid region.

Table 11. Summary of available homogeneous liquid density data along with the temperature and pressure ranges and their overall average absolute relative deviations (AAD) with respect to the equation of state. For a better overview, data sets with fewer than 5 data points are summarized and labeled as “All authors with fewer than five data points” in the last line of the table.

| Author | Year | N | T / K | p / MPa | AAD / % |
| --- | --- | --- | --- | --- | --- |
| Atilhan and Aparicio53 | 2013 | 126 | 278 - 359 | 0.1 - 60.0 | 0.043 |
| Bajić et al.54 | 2013 | 10 | 288 - 334 | 0.101325 | 0.022 |
| Bridgman52 | 1932 | 42 | 273 - 369 | <0.1 - 1177 | 1.2 |
| Domańska et al.55 | 2014 | 9 | 313 - 354 | 0.1 | 0.032 |
| Garber et al.56 | 1970 | 5 | 293.14 | 0.101325 | 0.16 |
| George and Sastry57 | 2003 | 5 | 298 - 339 | 0.101325 | 0.023 |
| Geyer et al.58 | 2000 | 5 | 278 - 319 | 0.101325 | 0.064 |
| Geyer et al.59 | 2001 | 12 | 288 - 309 | 0.1 - 60.0 | 0.026 |
| Guignon et al.51 | 2010 | 8 | 288 - 289 | 0.1 - 350 | 0.24 |
| Jiménez and Martinez60 | 2005 | 5 | 293 - 314 | 0.101325 | 0.017 |
| Jones and Tamplin61 | 1952 | 41 | 273 - 314 | 0.101325 | 0.030 |
| Khattab et al.62 | 2013 | 7 | 293 - 323 | 0.101325 | 0.26 |
| Krishna et al.63 | 2015 | 6 | 298 - 324 | 0.101325 | 0.028 |
| Li et al.64 | 2007 | 8 | 298 - 334 | 0.101325 | 0.052 |
| Li et al.65 | 2008 | 8 | 293 - 329 | 0.101325 | 0.036 |
| Ling et al.66 | 2011 | 6 | 298 - 324 | 0.101325 | 0.035 |
| Ling et al.67 | 2016 | 6 | 293 - 319 | 0.101325 | 0.042 |
| Makarov et al.68 | 2016 | 15 | 274 - 334 | 0.1 | 0.060 |
| Marchetti et al.69 | 2000 | 19 | 263 - 354 | 0.101325 | 0.036 |
| Nain70 | 2007 | 6 | 293 - 319 | 0.101325 | 0.049 |
| Olson and Cordray71 | 1992 | 7 | 273 - 419 | 0.101325 | 0.061 |
| Pal et al.72 | 2016 | 6 | 293 - 319 | 0.1 | 0.043 |
| Ponedelnikova and Tarasova73 | 1954 | 12 | 258 - 364 | 0.101325 | 0.066 |
| Rane et al.74 | 2016 | 6 | 293 - 319 | <0.1 | 0.022 |
| Romero et al.75 | 2008 | 6 | 283 - 309 | <0.1 | 0.037 |
| Sadykov et al.76 | 1974 | 16 | 302 - 453 | 0.101325 | 0.24 |
| Sagdeev et al.77 | 2017 | 57 | 293 - 453 | <0.1 - 246 | 0.20 |
| Saleh et al.78 | 1999 | 5 | 303 - 324 | 0.101325 | 0.077 |
| Sampson et al.40 | 2019 | 100 | 272 - 393 | 5.0 - 91.4 | 0.026 |
| Soldatović et al.79 | 2016 | 7 | 293 - 324 | 0.1 | 0.021 |
| Sun and Teja80 | 2004 | 8 | 298 - 441 | 0.1 - 2.2 | 0.14 |
| Timmermans and Hennaut-Roland81 | 1955 | 7 | 273 - 304 | 0.101325 | 0.021 |
| Tsai et al.82 | 2009 | 5 | 303 - 344 | 0.101325 | 0.059 |
| Vinogradov and Shakhparonov83 | 1984 | 7 | 243 - 333 | 0.1 | 0.027 |
| Zarei et al.84 | 2008 | 7 | 293 - 344 | <0.1 | 0.024 |
| Zarei et al.85 | 2013 | 6 | 293 - 334 | 0.101325 | 0.021 |
| Zemánková et al.86 | 2013 | 5 | 283 - 314 | 0.101325 | 0.024 |
| Zhuravlev87 | 1992 | 19 | 243 - 423 | 0.101325 | 0.22 |
| Zhuravlev et al.88 | 1985 | 13 | 243 - 364 | 0.101325 | 0.066 |
| Zivković et al.89 | 2014 | 8 | 288 - 324 | 0.101325 | 0.019 |
| Zorębski et al.27 | 2008 | 73 | 283 - 364 | 0.1 - 100 | 0.018 |
| All authors with fewer than five data points |  | 138 | 273 - 358 | <0.1 - 0.1 | 0.11 |

All data lie within (243 and 453) K, with pressures up to 1177 MPa. Without considering the data of Bridgman,52 density data are limited to a maximum pressure of 350 MPa.51 Due to the thermal decomposition of propylene glycol, thermodynamic properties at elevated temperatures (a rather broad temperature range of 200 K below the critical point) have not been experimentally investigated. This fact as well as the imprecise knowledge of the critical point was a challenge during the fitting procedure.

The data of Sampson et al.40 were specifically measured for the development of the present equation of state. With 100 state points in a temperature range of (272 to 393) K and pressures up to 92 MPa, it is the second most comprehensive data set. The measurements were carried out with a commercial high-pressure vibrating-tube densimeter (VTD) of the type DMH HPM manufactured by Anton Paar. The vibrating-tube measurement is based on the relation between the density and the vibration period of a U-shaped tube, which is measured by a frequency counter according to Kayukawa et al.90 Before the actual measurement, the VTD was calibrated at temperatures between (283 and 383) K with pressures up to 98 MPa. The substances used for the calibration were water and helium, which are typical compounds for this purpose. The liquid-phase densities of propylene glycol and water are similar, whereas helium is used as a fixed point at comparatively low densities. From two underlying models of Outcalt and McLinden91 and May et al.,92 a very accurate calibration correlation was obtained. Sampson et al.40 estimated the relative combined expanded uncertainty to be within 0.15 % (1.56 kg m–3), considering uncertainties of temperature, pressure, oscillation period, and calibration, as well as sample impurities of 99.5 mol. % for a degassed sample. Each single data point is represented by the equation of state with a deviation of less than 0.05 % (cf. Fig. 6), which is well within the specified uncertainties. In fact, the uncertainty estimate of the measurements seems to be very conservative to us.

Fig. 6. Deviations of homogeneous liquid density data from the equation of state for selected data sets in selected temperature ranges as a function of the logarithmic pressure.

For overlapping temperature ranges, the data set of Sampson et al.40 shows good agreement with another comprehensive data set obtained by Zorębski et al.27 These data are derived from accurate speed-of-sound measurements presented in the same paper. The procedure is based on the approach of Sun et al.93 When performed correctly, this leads to very accurate results, sometimes better than it would be possible with direct density measurements.93–95 The data set includes 73 data points in a temperature range of (283 to 364) K with fairly high pressures up to 100 MPa and the respective uncertainties are claimed to be 0.02 %. The deviation of the data from the equation of state are within this range for most state points within (298 to 364) K. At lower temperatures, deviations increase up to 0.06 %.

The homogeneous density measurements of Bridgman52 cover pressures of up to 1177 MPa. According to Bridgman,52 the measured densities are subject to an error in connection with thermal expansion, which, however, is negligibly small taking other uncertainties like impurities of the sample into account. The data were not used for the development of the equation of state but to verify its extrapolation behavior. The deviations from the equation of state are less than 1.6 % (cf. Fig. 7).

Fig. 7. Deviations of homogeneous liquid density data of Bridgman52 from the equation of state as a function of the logarithmic pressure.

Figure 8 shows the deviations of homogeneous liquid density data at atmospheric pressure. Showing good agreement with the data set of Sampson et al.40 at elevated pressures, the data set of Zorębski et al.27 is considered as a basis for analysis at atmospheric pressure. Moreover, their data are in good agreement with those obtained by Zarei et al.84 and Orge et al.96 The seven data points of Zarei et al.84 deviate by less than 0.034 % from the equation, which is well in agreement with the deviations of Zorębski et al.27 (0.06 %) in the corresponding temperature range. Zarei et al.84 used an Anton Paar DMA 4500 with a certified precision of 0.005 g dm−3 and an automatic viscosity correction, but no information on a combined uncertainty is given. The data set of Bajić et al.54 shows conforming data points to that of Zorębski et al.27 within a temperature range of (288 to 334) K at atmospheric pressure (cf. Fig. 8). They used a similar apparatus and the same calibration procedure as Zarei et al.,84 while claiming a quite optimistic experimental uncertainty of 0.01 kg m-3 (0.001 %). The measurements are represented with the equation of state within deviations of less than 0.032 %.

Fig. 8. Deviations of homogeneous liquid density data from the equation of state at atmospheric pressure as a function of temperature.

In Figure 8, curvature can be observed in the deviations of experimental data from the equation of state. Nonetheless, the deviations from the equation of state of the majority of the data sets are still within 0.05 % and, thus, represented within a reasonable range of uncertainty. In the course of the fitting process, the curvature was accepted in order to achieve a better representation of the data in the low temperature region (T < 270 K). A compromise was made so that the data of Vinogradov et al.83 and Zhuravlev et al.87,88 are represented within 0.07 %. However, to give a more reliable statement to define which low-temperature data should be fitted, further experimental investigations are required.

Based on the discussion above, uncertainties of densities calculated with the present equation of state at atmospheric pressure are estimated to be 0.06 % at temperatures between (270 and 380) K. At lower temperatures, uncertainties increase up to 0.1 %. The uncertainty of calculated densities at elevated pressures is estimated to be 0.06 % between (270 and 460) K and pressures up to 100 MPa based on the data of Sampson et al.40 and Zorębski et al.27

## Comparisons of vapor-liquid equilibrium data

In addition to a close investigation of the homogeneous liquid density data sets, several vapor pressure as well as saturated liquid density data sets are compared with respect to their representation with the equation of state. The collected data sets, including 246 vapor pressure and 17 saturated liquid density measurements, are presented in Table 12.

Table 12. Summary of vapor-pressure (pv) and saturated liquid density (ρ′) data along with the temperature ranges and their average absolute relative deviations (AAD) with respect to the equation of state.

|  |  |  |  | AAD / % |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Author | Year | N | T / K | LTa | MTa | HTa | Overall |
| pv |  |  |  |  |  |  |  |
| Chyliński et al.97 | 2004 | 8 | 393 - 424 | 3.8 | 4.2 |  | 4 |
| Fendu and Oprea98 | 2014 | 16 | 350 - 426 | 1.9 | 0.43 |  | 1.4 |
| Giles et al.99 | 1997 | 2 | 373 - 454 | 2.9 | 2.7 |  | 2.8 |
| Horstmann et al.100 | 2001 | 2 | 338 - 349 | 26 |  |  | 26 |
| Jones and Tamplin61 | 1952 | 15 | 293 - 434 | 2.7 | 0.42 |  | 2.2 |
| Kundu et al.101 | 1970 | 9 | 278 - 319 | 3.5 |  |  | 3.5 |
| Marsden et al.102 | 1954 | 10 | 318 - 462 | 11 | 0.78 |  | 7.2 |
| Mathuni et al.103 | 2011 | 12 | 378 - 461 | 2.1 | 0.67 |  | 1 |
| Nicolae and Oprea104 | 2014 | 15 | 350 - 421 | 1.9 | 0.48 |  | 1.6 |
| Puck et al.105 | 1946 | 5 | 298 - 353 | 11 |  |  | 11 |
| Rane et al.74 | 2016 | 17 | 389 - 444 | 5.4 | 5.6 |  | 5.6 |
| Riddick et al.106 | 1955 | 7 | 298 - 462 | 4.8 | 0.78 |  | 2.5 |
| Sokolov et al.107 | 1972 | 5 | 356 - 462 | 5.7 | 0.15 |  | 3.5 |
| Steele et al.41 | 2002 | 20 | 365 - 496 | 3.2 | 2.4 |  | 2.7 |
| Stull et al.108 | 1947 | 10 | 318 - 462 | 11 | 0.78 |  | 7.2 |
| Verevkin109 | 2004 | 14 | 283 - 332 | 7.4 |  |  | 7.4 |
| Verevkin et al.110 | 2009 | 19 | 293 - 330 | 7.5 |  |  | 7.5 |
| Wilding et al.45 | 1991 | 3 | 348 - 474 | 6.6 | 2.6 |  | 5.3 |
| Wilson et al.111 | 1989 | 2 | 273 - 313 | 45 |  |  | 45 |
| Xie and Chen112 | 1993 | 14 | 358 - 460 | 35 | 12 |  | 20 |
| Zhang et al.113 | 2013 | 21 | 390 - 461 | 2.2 | 0.68 |  | 1 |
| All authors with only one data point |  | 20 | 459 - 463 |  | 1.9 |  | 1.9 |
| ρ' |  |  |  |  |  |  |  |
| Horstmann et al.100 | 2001 | 1 | 298.15 | 0.026 |  |  | 0.026 |
| Steele et al.41 | 2002 | 6 | 323 - 449 | 0.068 | 0.26 |  | 0.13 |
| Wilding et al.45 | 1991 | 1 | 298.00 | 0.32 |  |  | 0.32 |
| Zander et al.114 | 1882 | 9 | 288 - 451 | 4.8 | 18 |  | 11 |
| a LT: T/Tc < 0.6; MT: 0.6 ≤ T/Tc ≤ 0.98; and HT: T/Tc > 0.98 |  |  |  |  |  |  |  |

The vapor-pressure data cover a temperature range of (273 to 496) K. Most of them are limited to fairly small vapor pressures from roughly (3.4×10–6 to 0.27) MPa. This has to be considered when assessing the percentage deviation of the data as shown in Fig. 9. Similar to the density measurements, there is a lack of data in the range of 200 K below the critical point due to thermal decomposition of propylene glycol.

Nicolae and Oprea104 and Fendu and Oprea98 provide vapor-pressure measurements between (350 and 426) K. Both data sets were measured with the same static apparatus, consisting of an equilibrium cell, which is connected with a U-shaped tube and heated by a thermostatic oil bath. Fendu and Oprea98 measured vapor-liquid equilibrium data of propylene glycol mixtures and for the pure substances as well. In order to validate their experimental procedure, they refer to the data of Nicolae and Oprea104 obtained for pure propylene glycol, which are in very good agreement with the recent data of Fendu and Oprea.98 However, with the exact same apparatus, high consistency is to be expected. For the given temperature range, both data sets contain corresponding vapor-pressure data between (0.8 and 25) kPa. The reported standard uncertainties of (0.012 and 0.034) kPa ((0.028 to 0.29) % for k = 2, respectively) are misleading because they are only related to the reproducibility of three repetitive measurements at each state point. Combined uncertainties are expected to be higher. Figure 9 shows that the data of Nicolae and Oprea104 and Fendu and Oprea98 deviate from the present equation of state by (2.9 and 2.7) %, respectively.

In the vicinity of the normal boiling point, a cluster of single state points from 20 publications is available (cf. Table 12). Dean42 reports a normal-boiling-point temperature of 461.15 K, which deviates by –0.074 K (0.23 %) from the calculated normal-boiling-point temperature in this work. Furthermore, the temperature of Marsden et al.102 at 461.35 K, which is highly referenced in the literature, agrees within +0.125 K (0.38 %) with the equation.

One of the most comprehensive vapor-pressure data sets was obtained by Steele et al.41 The measurements were carried out in a twin ebulliometric apparatus covering a temperature range from (365 to 496) K. They claim rather small uncertainties between 0.003 kPa (0.015 %) and 0.02 kPa (0.007 %) at higher temperatures. However, the data significantly deviate from those of Nicolae and Oprea104 and other researchers, e.g., Zhang et al.113 and Mathuni et al.103 Therefore, they were not considered in the fitting procedure, but are still reproduced within 3.8 %.

Fig. 9. Deviations of all available vapor-pressure data from the equation of state as a function of temperature. The bottom panel depicts the same data with a higher resolution of the y-axis.

The data set of Kundu et al.101 contains measurements in the low temperature region and yields correspondingly low vapor pressures from (2.7 to 90) Pa. These data agree with the measurements of Jones and Tamplin61 and they connect appropriately to the data of Nicolae and Oprea104 at higher temperatures. Therefore, these data were preferred over those of Verevkin,109 which exhibit a contradictory trend. Percentage deviation of the data of Kundu et al.101 amount up to 7 %.

Due to the numerically small values of vapor pressures calculated with the present equation of state, percentage uncertainties are higher than for other fluids. They are estimated to be 5 % (k = 2) between (290 and 500) K and are expected to increase at lower temperatures.

## Comparisons of caloric properties

In comparison to the thermal properties, the caloric data are rather limited, including 172 speed-of-sound (Table 13) and 73 isobaric heat-capacity (Table 14) data points.

The temperature and pressure ranges of all available speed-of-sound data are covered by the data sets of Dávila et al.16 and Zorębski et al.27 The two sets together include measurements within a temperature range of (292 to 354) K with pressures up to 102 MPa. The data show excellent agreement at moderate pressures. Discrepancies can be observed at higher pressures (p > 10 MPa, cf. Fig. 10). To clarify this situation, new measurements were performed in this work, which perfectly agree with the measurements of Dávila et al.16

Table 13. Summary of all liquid-phase speed-of-sound data along with the temperature and pressure ranges and their average absolute relative deviations (AAD) with respect to the equation of state.

| Author | Year | N | T / K | p / MPa | AAD / % |
| --- | --- | --- | --- | --- | --- |
| Dávila et al.16 | 2016 | 28 | 303 - 354 | <0.1 - 30.2 | 0.007 |
| George and Sastry57 | 2003 | 5 | 298 - 339 | 0.101325 | 0.91 |
| Grzybkowski and Warmińska115 | 2016 | 1 | 298.15 | 0.1 | 0.032 |
| Kishimoto and Nomoto116 | 1954 | 6 | 288 - 314 | 0.101325 | 0.12 |
| Krishna et al.63 | 2015 | 6 | 298 - 324 | 0.101325 | 0.15 |
| Krivokhizha and Fabelinskiǐ117 | 1966 | 10 | 246 - 294 | 0.101325 | 4.9 |
| Kushare et al.118 | 2008 | 1 | 298.15 | 0.101325 | 0.15 |
| Latha et al.119 | 2015 | 4 | 303 - 319 | 0.101325 | 0.12 |
| Marks120 | 1967 | 3 | 273 - 354 | 0.101325 | 0.14 |
| Nain121 | 2008 | 6 | 293 - 319 | 0.101325 | 0.52 |
| Pal et al.72 | 2016 | 6 | 293 - 319 | 0.1 | 0.11 |
| Palani and Geetha122 | 2009 | 3 | 303 - 313 | 0.101325 | 0.24 |
| Ponedelnikova and Tarasova73 | 1954 | 12 | 258 - 364 | 0.101325 | 0.33 |
| Sastry and Patel123 | 2003 | 2 | 298 - 309 | 0.101325 | 1.5 |
| This work | 2021 | 24 | 293 - 354 | 0.5 - 20.0 | 0.009 |
| Tsierkezos and Palaiologou124 | 2009 | 1 | 298.15 | 0.101325 | 0.018 |
| Zorębski et al.27 | 2008 | 45 | 292 - 314 | 0.1 - 102 | 0.022 |

The data of Zorębski et al.27 contain 45 state points in a very narrow temperature range of (292 to 314) K from ambient pressure up to 102 MPa. They claim an uncertainty of 1 m s−1, which corresponds to a percentage deviation of 0.067 % over the entire temperature range. Excluding the outlier at 15.2 MPa and 293 K, the data set of Zorębski et al.27 is well represented by the equation of state with deviations of less than 0.051 %.

Fig. 10. Deviations of the available speed-of-sound data in the liquid phase from the equation of state in selected temperature ranges as a function of the logarithmic pressure along with the speed-of-sound data measured in this work.

The measurements of Dávila et al.16 comprise 28 data points, from ambient pressure to 30.2 MPa in a temperature range of (303 to 354) K. The experimental uncertainty is stated to be 0.02 %. Both measurements were conducted with the ultrasonic pulse-echo-overlap method as already explained in detail in Sec. 3.1. However, Dávila et al.16 used a sample with higher purity (99.87 mol. %) than Zorębski et al.27 (99 mol. %) and, therefore, report a smaller experimental uncertainty. Except for one state point at atmospheric pressure, all data are represented within the experimental uncertainty. At ambient temperatures of (303 and 313) K, the data points of Dávila et al.16 and Zorębski et al.27 match within 0.009 percentage points (cf. Fig. 11), which shows the consistency of the data sets in that specific region. However, Fig. 10 shows that at pressures above approximately 10 MPa, the two data sets differ from each other and the data of Zorębski et al.27 slightly scatter. Therefore, measurements were carried out in this work to clarify this situation. For details see Sec. 3. These measurements prove the data of Dávila et al.16 to be more accurate. Therefore, our measurements were prioritized during the fitting procedure and are reproduced within 0.021 %, which is well within the experimental uncertainty.

Fig. 11. Deviations of the available speed-of-sound data in the liquid phase from the equation of state at atmospheric pressure as a function of temperature.

Figure 11 illustrates percentage deviations of experimental atmospheric speed-of-sound data from the present equation of state. The data set of Zorębski et al.27 is in very good agreement with two independent, single-point measurements of Grzybkowski and Warmińska115 and Tsierkezos and Palaiologou124 at 298.15 K. Grzybkowski and Warmińska115 claim an uncertainty of 0.15 m s−1, which corresponds to a percentage deviation of 0.01 %. It was measured with the pulse-echo method, with a sample of 99.5 mol. % purity. The measurement of Tsierkezos and Palaiologou124 was carried out with a similar purity, but is associated with a higher uncertainty (1 m s−1) due to a different measurement technique. They obtained the speed of sound with an Anton Paar (type: DSA 48) sound analyzer. Usually, this technique has high precision, but also requires complicated calibration procedures, which can cause considerable uncertainties according to Dzida et al.125

Based on the data measured in this work and of Dávila et al.,16 uncertainties of speed-of-sound values calculated with the present equation of state are 0.03 % (k = 2) between (290 and 360) K and a maximum pressure of 30 MPa. At higher pressures up to 100 MPa, uncertainties are estimated to be 0.07 % in the same temperature range based on the data of Zorębski et al.27

The available isobaric heat-capacity data are listed in Table 14 and their deviations are shown in Fig. 12. All data were measured at atmospheric pressure and temperatures between (274 and 354) K, except for the data set of Zorębski et al.,27 which provides data for the isobaric heat capacity at pressures up to 100 MPa.

Table 14. Summary of all isobaric heat-capacity data along with the temperature range and their overall average absolute relative deviations (AAD) with respect to the equation of state.

| Author | Year | N | T / K | p / MPa | AAD / % |
| --- | --- | --- | --- | --- | --- |
| Dean42 | 1999 | 1 | 298.15 | 0.101325 | 0.66 |
| Li et al.126 | 2009 | 6 | 303 - 354 | 0.101325 | 0.66 |
| Parks and Huffman127 | 1927 | 4 | 274 - 277 | 0.101325 | 0.61 |
| Pietrzak and Łudzik128 | 2015 | 1 | 298.15 | 0.1 | 1.1 |
| Riddick et al.129 | 1986 | 1 | 293.14 | 0.101325 | 0.75 |
| Zemánková et al.86 | 2013 | 5 | 283 - 314 | 0.101325 | 0.93 |
| Zorębski et al.27 | 2008 | 55 | 293 - 314 | 0.1 - 100 | 0.29 |

The 55 data points of Zorębski et al.27 were derived from speed-of-sound measurements at five isotherms covering a temperature range from (293 to 313) K. They used their speed-of-sound measurements to calculate the changes in isobaric heat capacity with varying pressure at constant temperature. As introduced by Sun et al.93 the actual calculations of the isobaric heat-capacity data were performed with a polynomial, temperature-dependent fit based on smoothed out values for the measured speed of sound, temperature and pressure, as well as on reference values for the isobaric heat capacity at atmospheric pressure. Zorębski et al.27 state an uncertainty of 0.3 % for the derived heat capacities of propylene glycol. At (303, 313) K, and atmospheric pressure the corresponding data points of Zorębski et al.27 and Li et al.126 are in good agreement. The data of Zorębski et al.27 deviate by less than 0.4 % from the equation of state at atmospheric pressure, which increases up to 0.75 % at the maximum pressure of 100 MPa. A more accurate representation of the data was not possible without deteriorating the speed-of-sound data of Zorębski et al.27

The data set of Li et al.126 contains isobaric heat-capacity data in a temperature range from (303 to 354) K. They used a differential scanning calorimeter (DSC) to measure the isobaric heat capacity of propylene glycol as well as several mixtures of glycols, water, and salts. They claim an uncertainty of 0.015 kJ kg−1 K−1, which corresponds to an uncertainty of (0.5 to 0.6) %. All data obtained by Li et al.126 are represented within 1.0 % with the equation of state. Again, a better representation of the data was not possible without degrading the speed-of-sound data.

Zemánková et al.86 provide the third comprehensive data set of isobaric heat capacity for propylene glycol. It covers a temperature range from (283 to 313) K at atmospheric pressure and was carried out with a Micro DSC III (Setaram) differential scanning calorimeter and a 99.5 mass % pure probe. All data are represented within their estimated relative uncertainty of 0.5 % and are in good agreement with data reported by Dean42 and Riddick129 at (298 and 293) K, respectively.

Fig. 12. Deviations of all available liquid-phase isobaric heat-capacity data from the equation of state as a function of temperature.

The data set obtained by Parks and Huffman127 contains 39 data points that are not entirely included in the analysis here, since data are predominantly measured within the glassy state. The four liquid data points at (275 to 277) K are represented within less than 0.8 % with the equation of state.

Since the heat-capacity data at atmospheric pressure scatter and even exhibit different trends (cf. Fig. 12), it is difficult to reliably state an uncertainty of values calculated with the present equation of state. Therefore, it is conservatively assessed to be 1.5 % (k = 2) within a temperature range of (270 to 360) K. Uncertainties at elevated pressures might be 1 % based on the data of Zorębski et al.27 However, additional measurements are required to prove this statement.

# Extrapolation and Physical Behavior

In addition to the precise representation of the experimental data, the extrapolation behavior must also be considered when assessing the quality of the equation of state. To ensure the physically correct behavior of the equation in fluid state regions at either very high or low temperature and pressure and close to the critical point, a smooth and steady course of constant properties in those regions is very important. In the following analysis, the behavior of various thermal and caloric properties of propylene glycol is investigated. The fluid regions with strong associative behavior are expected to show deviating shapes of constant property lines, comparable to those of other strongly associating fluids, e.g., heavy water.130 These peculiarities are predominantly observed in the consideration of caloric properties at low temperatures. Reasonable extrapolation behavior over a wide fluid range is of particular importance for propylene glycol. The relatively narrow region covered by experimental data can be extended in order to maintain a wider range of accessible fluid states. This is important when considering propylene glycol, e.g., as a hydrate inhibitor in mixture models. Mixture models are often evaluated in regions outside of the range of validity of the pure compounds. For asymmetric mixtures such as propylene glycol and carbon dioxide, correct extrapolation behavior of the pure-fluid EOS is mandatory.

The shape of the “ideal curves” (cf. Span and Wagner131) are fundamental to evaluate the extrapolation behavior of an EOS. Each ideal curve is usually defined in terms of derivatives of the compressibility factor with respect to temperature and density. The shapes of these ideal curves should be devoid of any bumps or discontinuities. The ideal curves calculated from the EOS of this work show reasonable behavior over the entire temperature and pressure range. As illustrated in Fig. 13, the coverage of underlying measurements compared to other fluids (e.g. CO2) is quite narrow for propylene glycol, and extrapolating outside this region requires that the ideal curves meet all criteria.

Figure 14 depicts two other important criteria for the thermal properties. In the top panel, a double-logarithmic pressure-density diagram with selected isotherms up to 106 K is shown. At high pressures, isotherms should merge with increasing temperature without crossing each other. In the bottom panel, a temperature-density plot with selected isobars is presented. It shows a straight rectilinear diameter ρRD=ρ'+ρ''2 up to the critical point and a distinct saddle point of the critical isobar at this point.

Another criterion for assessing the extrapolation behavior of the equation of state can be derived from the shape of the second (B), third (C), and fourth (D) virial coefficients. Starting at low temperatures and negative infinity, all three virial coefficients have a positive slope and negative curvature until they cross the zero line. They exhibit a maximum and approach zero without becoming negative again. The maximum of the third and fourth virial coefficients are expected to be around the critical temperature for simple fluids such as noble gases, cf., Thol et al.132 However, this might not be true for associating fluids such as propylene glycol, water,26 or heavy water.130 The maxima values are slightly shifted here. The theoretical behavior of the fourth virial coefficient is defined by a second, smaller maximum at higher temperatures as shown by Thol et al.,132 which is fulfilled by the equation of state (cf. Fig. 15).

In the vicinity of the critical region, analytical equations show limitations for the calculation of thermal and caloric properties due to the mathematical structure. Moreover, measurements in this region are associated with high uncertainties, if available at all. According to Span,29 the theoretical behavior of caloric properties close to and at the critical point are related to the pressure derivatives. The derivative ∂p∂ρT becomes zero and ∂p∂Tρ reaches a finite value at the critical point, which yields for example a diverging isobaric heat capacity. However, zero sound speed and infinite isochoric heat at the critical point cannot be obtained with the analytical equations, as currently used in all EOS developments due to the mathematical structure. This would require the use of non-analytical terms such as applied to the EOS of carbon dioxide133 and water,26 which is not possible here due to the restricted data base. Nonetheless, the equation should exhibit distinct minima/maxima at the critical point. The present EOS includes two Gaussian bell-shaped terms to approach the desired behavior of the speed of sound and the residual isochoric heat capacity (see Fig. 16 and 17) in the critical region.

Fig. 13. Ideal curves for the equation of state for propylene glycol (PGC) in terms of the reduced temperature T/Tc and pressure p/pc: Vapor-pressure curve (pv), Boyle curve (BL), Joule-Thomson inversion curve (JT), Joule inversion curve (JI), and the ideal curve (ID). The dashed lines mark the regions covered by experimental data for PGC and CO2.

Fig. 14 p,ρ-diagram along isotherms up to 106 K (top).ρρ T,ρ-diagram showing the phase boundaries, the rectilinear diameter, and the critical isobar (bottom).

Fig. 15 Second (B), third (C), and fourth (D) viral coefficients. The dashed lines indicate the critical temperature.

Figure 16 shows the speed of sound as a function of temperature along selected isobars. As expected, the liquid and vapor saturation line merge in a distinct minimum. The three most comprehensive data sets for this property of Zorębski et al.,27 Dávila et al.,16 and our results, covering isobars up to 100 MPa in the liquid phase, are included in the figure. Although these data cover only a small temperature and pressure range in the liquid phase, the equation shows a satisfactory course of the sound speed in the entire fluid region.

Fig. 16 Speed of sound along isobars up to 120 MPa (in steps of 8 MPa) as a function of temperature. The three most comprehensive data sets are plotted for a better assessment of the data situation. The dashed line shows the lower temperature limit of the equation of state at the triple-point temperature.

Fig. 17 Residual isochoric heat capacity along isobars up to 50 MPa (in steps of 5 MPa) as a function of temperature. The dashed line shows the lower temperature limit of the equation of state at the triple-point temperature.

The residual isochoric heat capacity is closely related to the speed of sound. Accompanied by the distinct minimum of sound speed, the saturated liquid and vapor curves of the residual isochoric heat capacity merge in a distinct maximum at the critical point. In the vicinity of the critical point, the steep increasing saturation lines should cross each other and meet at the critical point, which is fulfilled by the equation of state (cf. Fig. 17).The conspicuous behavior between the glass-transition temperature of 169.15 K and the triple-point temperature at 242.8 K is due to a distinct change in the internal energy/enthalpy and, thus, in heat capacity in the transitional region between the glassy and the liquid state. This can also be observed for related properties such as the speed of sound (cf. Fig. 16).

Figure 18 shows the phase identification parameter134 over a broad temperature range along various isobars for the present equation of state. Due to its high dependency on partial derivatives of pressure, volume, and temperature, the phase identification parameter is an important property when assessing the quality of the equation of state. Except for the association effects in the same temperature range as for the isochoric heat capacity, the saturated liquid line should show all positive derivatives and a distinct maximum at the critical point, while the saturated vapor line should exhibit an opposing behavior and reach a minimum.

Fig. 18 Phase identification parameter as a function of temperature along selected isobars.

The Grüneisen parameter establishes the connection between thermal and caloric properties and is, thus, another important criterion in assessing the extrapolation behavior of the equation. Similar to the speed of sound, it should exhibit merging saturation lines with a distinct minimum at the critical point (cf. Fig. 19). At low temperatures, the association effect becomes obvious.

Fig. 19 Residual Grüneisen parameter as a function of temperature along selected isobars.

The analysis of these plots show that the equation exhibits correct physical and extrapolation behavior and can, thus, be applied to mixture models.

# Conclusion

A fundamental equation of state in terms of the Helmholtz energy with the independent variables temperature and density was developed for propylene glycol. Accurate speed-of-sound measurements were carried out with the pulse-echo technique between (293 and 354) K with a maximum pressure of 20 MPa in order to validate experimental data from the literature. The experimental uncertainty in the speed-of-sound measurements is estimated to be 0.03 % (k = 2).

There were some particular challenges during the development of the equation of state, especially with the inadequate data situation. Propylene glycol exhibits quite strong association in the liquid phase, which particularly changes the behavior of properties in the lower temperature region. Furthermore, due to thermal decomposition, a broad temperature range has not been investigated experimentally and the critical point is not well known. There is no reliable information available on the isobaric heat capacity of the ideal gas so that the ideal part of the equation of state had to be adjusted to liquid speed-of-sound and heat-capacity data.

The equation can be used to calculate all thermodynamic properties between the triple-point temperature 242.8 K and the maximum temperature of 680 K with pressures up to 350 MPa. Based on the experimental data, estimations on the uncertainties (k = 2) of the properties calculated with the equation of state are given in the corresponding parts of Sec. 9. Uncertainties of densities calculated with the present equation of state at atmospheric pressure are estimated to be 0.06 % at temperatures between (270 and 380) K and increase up to 0.1 % at lower temperatures. The uncertainty of calculated densities at elevated pressures is estimated to be 0.06 % between (270 and 460) K and pressures up to 100 MPa. Uncertainties with respect to calculated vapor pressures are 5 % between (290 and 500) K and are expected to increase at lower temperatures. Uncertainties of liquid-phase speeds of sound calculated with the present equation of state are 0.03 % between (290 and 360) K and a maximum pressure of 30 MPa. At higher pressures up to 100 MPa, uncertainties are estimated to be 0.07 %. Calculated isobaric heat-capacity data are assessed to be uncertain by 1.5 % within a temperature range of (270 to 360) K at atmospheric pressure, whereas it is 1 % at elevated pressures.

Since propylene glycol is considered as a component in various mixture applications, special attention was given to reliable physical behavior of the equation in regions where no experimental data are available.

Acknowledgement

The authors thank Dr. Ian H. Bell for providing the Python script available in the supplementary material.

Supporting Information Available

In the supplementary material, a fluid file for the application in the software packages TREND135 and REFPROP136 is provided (PROPYLENEGLYCOL.FLD). Furthermore, a stand-alone Python script is available for the calculation of common thermodynamic properties with the present equation of state for propylene glycol (propyleneglycol.py). Additional Python files for the use in CoolProp137 are also provided (PropyleneGlycol.json, test_CoolProp_PG.py).

The data that support the findings of this study are available within the article and its supplementary material.

# References

1  B. A. Stefl and K. F. George, Antifreezes and Deicing Fluids in Krik-Othmer Encyclopedia of Chemical Technology, edited by R. E. Kirk and D. F. Othmer (John Wiley & Sons), 2000, pp. 1–21.

2  A. Chapoy, R. W. Burgass, and B. Tohidi Kalorazi, Hydrate inhibition in propylene glycol and glycerol systems, paper presented at 8th International Conference on Gas Hydrates (China, Beijing, 2014).

3  J. Gernert and R. Span, J. Chem. Thermodyn. 93, 274 (2016).

4  R. Feistel and W. Wagner, J. Phys. Chem. Ref. Data 35, 1021 (2006).

5  A. Jäger and R. Span, J. Chem. Eng. Data 57, 590 (2012).

6  M. Trusler, J. Phys. Chem. Ref. Data 40, 043105 (2011).

7  V. Vinš, A. Jäger, R. Span, and J. Hrubý, Fluid Phase Equilib. 427, 268 (2016).

8  V. Vinš, A. Jäger, J. Hrubý, and R. Span, Fluid Phase Equilib. 435, 104 (2017).

9  A. Jäger, V. Vinš, R. Span, and J. Hrubý, Fluid Phase Equilib. 429, 55 (2016).

10  J. A. Ruddick, Toxicology and Applied Pharmacology 21, 102 (1971).

11  P. Boutron and A. Kaufmann, Cryobiology 16, 557 (1979).

12  N. H. Rhys, R. J. Gillams, L. E. Collins, S. K. Callear, M. J. Lawrence, and S. E. McLain, J. Chem. Phys. 145, 1 (2016).

13  W. L. Howard, J. Chem. Eng. Data 14, 129 (1969).

14  M. E. Wieser and M. Berglund, Pure Appl. Chem. 81, 2131 (2009).

15  E. Tiesinga, P. J. Mohr, D. B. Newell, and B. N. Taylor, The 2018 CODATA Recommended Values of the Fundamental Physical Constants (Web Version 8.1). Database developed by J. Baker, M. Douma, S. Kotochigova. Available at http://​physics.nist.gov​/​constants (National Institute of Standards and Technology, Gaithersburg, USA, 2020).

16  M. J. Dávila, H. Gedanitz, and R. Span, J. Chem. Thermodyn. 101, 199 (2016).

17  H. Gedanitz, M. J. Dávila, E. Baumhögger, and R. Span, J. Chem. Thermodyn. 42, 478 (2010).

18  K. Meier and S. Kabelac, Rev. Sci. Instrum. 77, 123903 (2006).

19  R. Wegge, Thermodynamic Properties of the (Argon + Carbon Dioxide) System: Instrument Development and Measurements of Density and Speed of Sound (Ruhr-Universität Bochum, 2016).

20  R. Wegge, M. Richter, and R. Span, J. Chem. Eng. Data 60, 1345 (2015).

21  F. H. Dubberke, E. Baumhögger, and J. Vrabec, Rev. Sci. Instr. 86, 054903 (2015).

22  G. R. Harris, J. Acoust. Soc. Am. 70, 186 (1981).

23  H. Gedanitz, Schallgeschwindigkeits- und Dichtemessungen in Fluiden (Ruhr-Universität Bochum, 2010).

24  V. A. Del Grosso and C. W. Mader, J. Acoust. Soc. Am. 52, 1442 (1972).

25  K. Fujii and R. Masui, J. Acoust. Soc. Am. 93, 276 (1993).

26  W. Wagner and A. Pruß, J. Phys. Chem. Ref. Data 31, 387 (2002).

27  E. Zorębski, M. Dzida, and M. Piotrowska, J. Chem. Eng. Data 53, 136 (2008).

28  K. Lucas, Applied Statistical Thermodynamics (Springer, Berlin, 1991).

29  R. Span, Multiparameter equations of state: An accurate source of thermodynamic property data (Springer, Berlin, 2000).

30  L. Haar, J. S. Gallagher, and G. S. Kell, A Thermodynamic Surface of Water: The Formulation and Computer Programs in Proceedings of the 8th Symposium on Thermophysical Properties, edited by J. V. Sengers (American Society of Mechanical Engineers, New York), 1982, pp. 298–302.

31  U. Setzmann and W. Wagner, J. Phys. Chem. Ref. Data 20, 1061 (1991).

32  K. Gao and E. W. Lemmon, Private Communication to T. Eisenbach (2020).

33  E. Diaz, M. E. Sad, and E. Iglesia, ChemSusChem 3, 1063 (2010).

34  D. M. VonNiederhausern, G. M. Wilson, and N. F. Giles, J. Chem. Eng. Data 45, 157 (2000).

35  A. F. Kazakov, C. D. Muzny, V. Diky, R. D. Chirico, and M. Frenkel, Fluid Phase Equilib. 298, 131 (2010).

36  W. H. Carande, A. F. Kazakov, C. D. Muzny, and M. Frenkel, J. Chem. Eng. Data 60, 1377 (2015).

37  D. M. VonNiederhausern, L. C. Wilson, N. F. Giles, and G. M. Wilson, J. Chem. Eng. Data 45, 154 (2000).

38  K. G. Joback and R. C. Reid, Chem. Eng. Comm. 57, 233 (1987).

39  B. E. Poling, J. M. Prausnitz, and J. P. O'Connell, The Properties of Gases and Liquids 5th ed. (McGraw-Hill, Inc., New York, 2001).

40  C. C. Sampson, X. Yang, J. Xu, and M. Richter, J. Chem. Thermodyn. 131, 206 (2019).

41  W. V. Steele, R. D. Chirico, S. E. Knipmeyer, and A. Nguyen, J. Chem. Eng. Data 47, 689 (2002).

42  J. A. Dean, Lange's Handbook of Chemistry (McGraw-Hill, Inc., New York, 1999).

43  C. L. Yaws, Chemical Properties Handbook: Physical, Thermodynamic, Environmental, Transport, Safety, and Health Related Properties for Organic and Inorganic Chemicals (McGraw-Hill, Inc., New York, 1999).

44  R. W. Gallant, Physical Properties of Hydrocarbons (Gulf Publishing Company, Texas, 1968).

45  W. V. Wilding, L. C. Wilson, and G. M. Wilson, AlChE Data Series 1, 6 (1991).

46  VDI-Gesellschaft Verfahrenstechnik und Chemieingenieurwesen (GVC), VDI Wärmeatlas (Springer, Berlin, Heidelberg, 2013).

47  V. Diky, C. D. Muzny, A. Y. Smolyanitsky, A. Bazyleva, R. D. Chirico, J. W. Magee, Y. Paulechka, A. F. Kazakov, S. A. Townsend, E. W. Lemmon, M. Frenkel, and K. Kroenlein, ThermoData Engine (TDE) Version 10.2 (Pure Compounds, Binary Mixtures, Ternary Mixtures, and Chemical Reactions): NIST Standard Reference Database 103b (National Institute of Standards and Technology, Gaithersburg, USA, 2019).

48  V. Diky, R. D. Chirico, C. D. Muzny, A. F. Kazakov, K. Kroenlein, J. W. Magee, I. Abdulagatov, and M. Frenkel, J. Chem. Inf. Model. 53, 3418 (2013).

49  E. W. Lemmon and R. Span, J. Chem. Eng. Data 60, 3745 (2015).

50  M. Thol, G. Rutkai, A. Köster, F. H. Dubberke, T. Windmann, R. Span, and J. Vrabec, J. Chem. Eng. Data 61, 2580 (2016).

51  B. Guignon, C. Aparicio, and P. D. Sanz, J. Chem. Eng. Data 55, 3017 (2010).

52  P. W. Bridgman, Proc. Am. Acad. Arts Sci. 67, 1 (1932).

53  M. Atilhan and S. Aparicio, J. Chem. Thermodyn. 57, 137 (2013).

54  D. M. Bajić, G. R. Ivaniš, Z. P. Visak, E. M. Zivković, S. P. Šerbanović, and M. L. Kijevčanin, J. Chem. Thermodyn. 57, 510 (2013).

55  U. Domańska, P. Papis, J. Szydłowski, M. Królikowska, and M. Królikowski, J. Phys. Chem. B 118, 12692 (2014).

56  Y.N. Garber, L.F. Komarova, L.I. Aleinikova, and L.S. Fomina, Zh. Prikl. Khim. 43, 2658 (1970).

57  J. George and N. V. Sastry, J. Chem. Eng. Data 48, 1529 (2003).

58  H. Geyer, P. Ulbig, and M. Görnert, J. Chem. Thermodyn. 32, 1585 (2000).

59  H. Geyer, P. Ulbig, M. Görnert, and A. Susanto, J. Chem. Thermodyn. 33, 987 (2001).

60  J. Jiménez and F. Martinez, Rev. Colomb. Cienc. Quím. Farm. 34, 46 (2005).

61  W.S. Jones and W.S. Tamplin, Chapter 9. Physical Properties of Propylene Glycol in Glycols. American Chemical Society Monograph 114, edited by G.O. Curme Jr. (Reinhold Publishing Corporation, New York), 1952, pp. 210–240.

62  I. S. Khattab, F. Bandarkar, M. Khoubnasabjafari, and A. Jouyban, Arab. J. Chem. 10, 71 (2013).

63  T. S. Krishna, M. Gowri Sankar, K. R. Thomas, S. Govardhan Rao, and B. Munibhadrayya, J. Mol. Liq. 206, 350 (2015).

64  Q.-S. Li, M.-G. Su, and S. Wang, J. Chem. Eng. Data 52, 1141 (2007).

65  Q.-S. Li, Y.-M. Tian, and S. Wang, J. Chem. Eng. Data 53, 271 (2008).

66  J.-L. Ling, F. Cao, M. Xu, and L. Yu, J. Chem. Ind. Eng. China 62, 1191 (2011).

67  J.-L. Ling, L. Li, and J.-L. Yang, J. Chem. Eng. Chinese Univ. 30, 268 (2016).

68  D. M. Makarov, G. I. Egorov, and A. M. Kolker, J. Mol. Liq. 222, 656 (2016).

69  A. Marchetti, G. Pályi, L. Tassi, A. Ulrici, and C. Zucchi, J. Mol. Liq. 88, 183 (2000).

70  A. K. Nain, J. Chem. Thermodyn. 39, 462 (2007).

71  J. D. Olson and D. C. Cordray, Fluid Phase Equilib. 76, 213 (1992).

72  A. Pal, M. Saini, and B. Kumar, Fluid Phase Equilib. 411, 66 (2016).

73  E. G. Ponedelnikova and V. V. Tarasova, Dokl. Akad. Nauk SSSR 96, 1191 (1954).

74  N. V. Rane, A. Kumari, J. Soujanya, and B. Satyavathi, J. Chem. Thermodyn. 97, 142 (2016).

75  C. M. Romero, M. S. Páez, and D. Pérez, J. Chem. Thermodyn. 40, 1645 (2008).

76  A.K. Sadykov, D.I. Sagdeev, V.P. Brykov, and G.K. Mukhamedzyanov, Trudy Kazanskogo Khimiko-Tekhnologicheskogo Instituta 53, 98 (1974).

77  D.I. Sagdeev, M.G. Fomina, and I.M. Abdulagatov, Fluid Phase Equilib. 450, 99 (2017).

78  M. A. Saleh, S. Begum, S. K. Begum, and B. A. Begum, Phys. Chem. Liq. 37, 785 (1999).

79  D. A. Soldatović, J. M. Vuksanović, I. R. Radović, and M. L. Kijevčanin, J. Chem. Thermodyn. 102, 105 (2016).

80  T. Sun and A. S. Teja, J. Chem. Eng. Data 49, 1311 (2004).

81  J. Timmermans and Hennaut-Roland, J. Chim. Phys. 52, 223 (1955).

82  C.-Y. Tsai, A. N. Soriano, and M.-H. Li, J. Chem. Thermodyn. 41, 623 (2009).

83  A.N. Vinogradov and M.I. Shakhparonov, Viniti, Code 1479-78, 1 (1984).

84  H. A. Zarei, S. Asadi, and H. Iloukhani, J. Mol. Liq. 141, 25 (2008).

85  H. Zarei, S. A. Golroudbari, and M. Behroozi, J. Mol. Liq. 187, 260 (2013).

86  K. Zemánková, J. Troncoso, and L. Romaní, Fluid Phase Equilib. 356, 1 (2013).

87  V. I. Zhuravlev, Zh. Fiz. Khim. 66, 225 (1992).

88  V. I. Zhuravlev, V. A. Durov, T. M. Usacheva, and M. I. Shakhparonov, Zh. Obshch. Khim. 55, 992 (1985).

89  E. M. Zivković, D. M. Bajić, I. R. Radović, S. P. Šerbanović, and M. L. Kijevčanin, Fluid Phase Equilib. 373, 1 (2014).

90  Y. Kayukawa, M. Hasumoto, and K. Watanabe, Rev. Sci. Instr. 74, 4134 (2003).

91  S. L. Outcalt and M. O. McLinden, Ind. Eng. Chem. Res. 46, 8264 (2007).

92  E. F. May, W. J. Tay, M. Nania, A. Aleji, S. Al-Ghafri, and J. P. M. Trusler, Rev. Sci. Instr. 85, 095111 (2014).

93  T. F. Sun, C. A. Ten Seldam, P. J. Kortbeek, N. J. Trappeniers, and S. N. Biswas, Phys. Chem. Liq. 18, 107 (1988).

94  J. P. M. Trusler and E. W. Lemmon, J. Chem. Thermodyn. 109, 61 (2017).

95  W. Wagner and M. Thol, J. Phys. Chem. Ref. Data 44, 043102 (2015).

96  B. Orge, B. E. de Cominges, G. Marino, M. Iglesias, and J. Tojo, Phys. Chem. Liq. 39, 99 (2001).

97  K. Chyliński, Z. Fraś, and S. K. Malanowski, J. Chem. Eng. Data 49, 18 (2004).

98  E. M. Fendu and F. Oprea, Fluid Phase Equilib. 382, 244 (2014).

99  N. F. Giles, L. C. Wilson, G. M. Wilson, and W. V. Wilding, J. Chem. Eng. Data 42, 1067 (1997).

100  S. Horstmann, H. Gardeler, K. Fischer, F. Köster, and J. Gmehling, J. Chem. Eng. Data 46, 337 (2001).

101  K. K. Kundu, P. K. Chattopadhyay, D. Jana, and M. N. Das, J. Chem. Eng. Data 15, 209 (1970).

102  C. Marsden, Solvents and Allied Substances Manual (Cleaver-Hume Press LTD, London, 1954).

103  T. Mathuni, J.-I. Kim, and S.-J. Park, J. Chem. Eng. Data 56, 89 (2011).

104  M. Nicolae and F. Oprea, Fluid Phase Equilib. 370, 34 (2014).

105  T. T. Puck and H. Wise, J. Phys. Chem. 50, 329 (1946).

106  J. A. Riddick and E. E. Toops, Techn. Org. Chem. 7, 90 (1955).

107  N. M. Sokolov, L. N. Tsygankova, M. I. Shtrom, and N. M. Zhavoronkov, Sov. Chem. Ind. 48, 427 (1972).

108  D. R. Stull, Ind. Eng. Chem. 39, 517 (1947).

109  S. P. Verevkin, Fluid Phase Equilib. 224, 23 (2004).

110  S. P. Verevkin, V. N. Emel’yanenko, and G. Nell, J. Chem. Thermodyn. 41, 1125 (2009).

111  L. C. Wilson, W. V. Wilding, and G. M. Wilson, AlChE Symp. Ser. 85, 25 (1989).

112  R. Xie and C. Chen, J. Chem. Eng. Chinese Univ. 7, 267 (1993).

113  L. Zhang, W. Wu, Y. Sun, L. Li, B. Jiang, X. Li, N. Yang, and H. Ding, J. Chem. Eng. Data 58, 1308 (2013).

114  A. Zander, Justus Liebigs Ann. Chem. 214, 138 (1882).

115  W. Grzybkowski and D. Warmińska, J. Chem. Eng. Data 61, 2933 (2016).

116  T. Kishimoto and O. Nomoto, J. Phys. Soc. Jpn. 9, 1021 (1954).

117  S. V. Krivokhizha and I. L. Fabelinskii, Soviet Physics, JETP 23, 1 (1966).

118  S. K. Kushare, D. H. Dagade, and K. J. Patil, J. Chem. Thermodyn. 40, 78 (2008).

119  S. V. Latha, G. Little Flower, K. Rayapa Reddy, C. V. Nageswara Rao, and A. Ratnakar, J. Mol. Liq. 209, 153 (2015).

120  G. W. Marks, J. Acoust. Soc. Am. 41, 103 (1967).

121  A. K. Nain, J. Mol. Liq. 140, 108 (2008).

122  R. Palani and A. Geetha, Phys. Chem. Liq. 47, 542 (2009).

123  N. V. Sastry and M. C. Patel, J. Chem. Eng. Data 48, 1019 (2003).

124  N. G. Tsierkezos and M. M. Palaiologou, Phys. Chem. Liq. 47, 447 (2009).

125  M. Dzida, E. Zorębski, M. Zorębski, M. Zarska, M. Geppert-Rybczynska, M. Chorazewski, and I. Cibulka, Chem. Rev. 117, 3883 (2017).

126  C.-K. Li, A. N. Soriano, and M.-H. Li, Thermochim. Acta 487, 26 (2009).

127  G. S. Parks and H. M. Huffman, J. Phys. Chem. 31, 1842 (1927).

128  A. Pietrzak and K. Łudzik, Fluid Phase Equilib. 401, 56 (2015).

129  J. A. Riddick, W. B. Bunger, and T. K. Sakano, Techniques of Chemistry. 3. Physical Properties: Tabulations: Organic Solvents (John Wiley & Sons, 1986).

130  S. Herrig, M. Thol, A. H. Harvey, and E. W. Lemmon, J. Phys. Chem. Ref. Data 47, 043102 (2018).

131  R. Span and W. Wagner, Int. J. Thermophys. 18, 1415 (1997).

132  M. Thol, G. Rutkai, A. Köster, R. Lustig, R. Span, and J. Vrabec, J. Phys. Chem. Ref. Data 45, 023101 (2016).

133  R. Span and W. Wagner, J. Phys. Chem. Ref. Data 25, 1509 (1996).

134  G. Venkatarathnam and L. R. Oellrich, Fluid Phase Equilib. 301, 225 (2011).

135  R. Span, R. Beckmüller, S. Hielscher, A. Jäger, E. Mickoleit, T. Neumann, S. Pohl, B. Semrau, and M. Thol, TREND. Thermodynamic Reference and Engineering Data 5.0 (Lehrstuhl für Thermodynamik, Ruhr-Universität Bochum, Bochum, Germany, 2020).

136  E. W. Lemmon, I. H. Bell, M. L. Huber, and M. O. McLinden, NIST Standard Reference Database 23: Reference Fluid Thermodynamic and Transport Properties-REFPROP, Version 10.0 (National Institute of Standards and Technology, Gaithersburg, USA, 2018).

137  I. H. Bell, J. Wronski, S. Quoilin, and V. Lemort, Ind. Eng. Chem. Res. 53, 2498 (2014).